STATUTORY INSTRUMENTS

202X No. X

INFRASTRUCTURE PLANNING

# The Rampion 2 Offshore Wind Farm Order 20XX

Made	-	-	-	-	***

Laid before Parliament	***

Coming into force	***

Citation and commencement

This Order may be cited as the Rampion 2 Offshore Wind Farm Order 202X and comes into force on [ ].

Compensation measures

Schedule 17 (Compensation to protect the coherence of the National Site Network) has effect.

Signed by Authority of the Secretary of State for Energy, Security and Net Zero

Name

Address	Head of [  ]

Date	Department of Energy, Security and Net Zero

SCHEDULE 1	Article 3

AUTHORISED PROJECT

PART 1

THE AUTHORISED DEVELOPMENT

- A nationally significant infrastructure project as defined in sections 14 and 15 of the 2008 Act which is located in the English Channel approximately 13 kilometres to the south of the Sussex coast, comprising works to be carried out in the areas shown on the offshore works plans and onshore works plans—

Work No. 1—

an offshore wind turbine generating station which is to have a gross electrical output of over 100 megawatts and must comprise no more than 90 wind turbine generators, with each wind turbine fixed to the seabed by either monopile foundation or multileg foundation and including a connection to the subsea cable circuits comprising Work No. 2(a) or 2(b).

Work No. 2—

a network of subsea cable circuits between the wind turbine generators comprising Work No. 1;

a network of subsea cable circuits between the wind turbine generators comprising Work No. 1 and the offshore substations comprising Work No. 3(a) including one or more cable crossings; and

connections into the offshore substations comprising Work No. 3(a).

Work No. 3—

    - up to three offshore substations each fixed to the seabed by monopile foundation, or multileg foundation and including connection to the cable circuits comprising Work No. 2(b), cable circuits comprising Work No. 3(b) and cable circuits comprising Work No. 4; and

up to two offshore subsea cable circuits in total each connecting one offshore substation to another,

and associated development within the meaning of section 115(2) (development for which development consent may be granted) of the 2008 Act comprising—

Work No. 4—

up to four cable circuits in total between and connecting one or more of the offshore substations comprising Work No. 3(a) and the cable circuits comprising Work No. 5 laid on or beneath the seabed within the offshore Order limits seaward of MLWS.

Work No. 5—

    - up to four temporary horizontal directional drilling exit pits located seaward of MLWS; and

up to four cable circuits and associated ducts connecting the cable circuits comprising Work No. 4 and the cable circuits comprising Work No. 6 laid on or beneath the seabed or in ducts laid on or beneath the seabed within the offshore Order limits.

In the intertidal area

Work No. 6—

landfall connection works comprising up to four cable circuits and associated ducts laid underground by horizontal directional drilling within the Order limits seaward of MHWS and landward of MLWS connecting Work No. 5 and Work No. 7.

In the district of Arun, in the county of West Sussex

Work No. 7—

onshore connection works consisting of up to four cable circuits and associated cable ducts laid underground by horizontal directional drilling between and connecting Work No. 6 and Work No. 8.

Work No. 8—

onshore connection works at landfall consisting of—

    - up to 4 horizontal directional drilling launch and exit pits and associated cable ducts;

up to 4 transition joint bays; and

up to four cable circuits and associated cable ducts laid underground by open cut trenching, horizontal directional drilling, or other trenchless technology connecting Work No. 7 to Work No. 9.

In the districts of Arun, Horsham and Mid Sussex in the county of West Sussex, and within the South Downs National Park

Work No. 9—

onshore connection works consisting of—

    - up to four cable circuits and associated cable ducts laid underground either by open cut trenching, horizontal directional drilling or other trenchless technology;

horizontal directional drilling launch and exit pits and associated cable ducts or launch and exit pits associated with other trenchless technologies;

joint bays;

temporary construction consolidation sites, construction of a haul road and accesses;

installation of new land drainage infrastructure and modification of existing land drainage; and

the provision of operational access.

Work No. 10—

temporary construction compounds.

Work No. 11—

temporary soil storage area.

Work No. 12—

non-intrusive works for duct and cable installation preparation and stringing out.

Work No. 13—

temporary construction accesses including creation of visibility splays and vegetation clearance.

Work No. 14—

construction and operational accesses including creation of visibility splays and vegetation clearance.

Work No.15—

operational accesses including creation of visibility splays and vegetation clearance.

In the district of Horsham, in the county of West Sussex—

Work No. 16—

onshore substation and associated construction works including—

    - civil works to allow the installation of electrical equipment;

installation and commissioning of high voltage electrical equipment including switch gears and circuit breakers, transformers, reactive compensation equipment, high voltage cables and bus bars to connect electrical equipment;

construction of associated control buildings for the operation of the substation;

accesses connecting to Work No. 18;

surface water and drainage management measures;

boundary treatment works; and

landscaping.

Work No. 17—

environmental works to mitigate the impact of the substations comprising each of Work No. 16 and Work 20.

Work No. 18—

construction and operational access including drainage connecting Work No. 16 to the A272 including a new access junction.

In the districts of Mid Sussex and Horsham, in the county of West Sussex—

Work No. 19—

onshore cable installation and associated works connecting Work No. 16 to the extended National Grid substation at Bolney comprising Work No. 20 consisting of—

    - up to two cable circuits and associated cable ducts laid underground either by open cut trenching, horizontal directional drilling or other trenchless technology;

horizontal directional drilling launch and exit pits or launch and exit pits associated with other trenchless technologies and associated cable ducts;

transition joint bays;

temporary construction consolidation sites, construction of a haul road and accesses; and

installation of a new land drainage infrastructure and modification of existing land drainage and operational access.

Work No. 20—

an extension to the existing National Grid substation at Bolney for the connection of the cable circuits from the onshore substation to the National Grid substation including (but not limited to) cabling, cable sealing ends, circuit breakers, surge arrestors, dis-connectors, instrumentation-transformers, busbars and busbar clamp measuring equipment, other instrumentation and control equipment, relay marshalling rooms and electrical earthing works including associated connection works within the existing National Grid substation.

Further Works

Offshore

In connection with such Work Nos. 1 to 6 and to the extent that they do not otherwise form part of any such work, further associated development comprising such other works as may be necessary or expedient for the purposes of or in connection with the relevant part of the authorised project and which fall within the scope of the work assessed by the environmental statement, including—

    - scour protection around the foundations of the offshore structures;

cable protection measures;

vessels carrying out intrusive activities;

the removal of material from the seabed required for the construction of Work Nos. 1 to 5 and the disposal of inert material of natural origin within the Order limits produced during construction drilling, seabed preparation for foundation works, cable installation preparation such as sandwave clearance, boulder clearance, pre-trenching, horizontal directional drill arisings and excavation of exit pits;

removal of static fishing equipment; and

disposal of drill arisings in connection with any foundation drilling within the offshore Order limits,

and

Onshore

in connection with such Work Nos. 6 to 20 and to the extent that they do not otherwise form part of any such work, further associated development comprising such other works as may be necessary or expedient for the purposes of or in connection with the relevant part of the authorised development and which fall within the scope of the work assessed by the environmental statement, including—

    - works to secure vehicular and/or pedestrian means of access including the creation of new tracks, footpaths, and/or widening, creation of passing places, upgrades, creation of bellmouths, creation of temporary slip roads and improvements of existing tracks, footpaths and roads, including provision of visibility splays;

temporary construction access tracks or haul road;

bunds, embankments, swales, landscaping, fencing and boundary treatments and other means of enclosure;

vegetation clearance;

habitat creation and restoration;

joint bays, link boxes, cable ducts, cable protection, joint protection, manholes, marker posts, underground cable marker, tiles and tape and other works associated with cable laying;

works for the provision of apparatus including cabling, water and electricity supply works, foul drainage provision, surface water management systems and culverting including drainage mitigation works;

works to alter the position of apparatus, including mains, sewers, drains and cables;

works to alter the course of, or otherwise interfere with, non-navigable rivers, streams or watercourses;

landscaping and other works to mitigate any adverse effects of the construction, maintenance or operation of the authorised project;

works for the benefit or protection of land affected by the authorised development;

working sites in connection with the construction of the authorised development, construction lay down areas and compounds, storage compounds and their restoration;

car parking areas, welfare facilities, temporary offices and workshops;

spoil (including arisings from Work No. 6) and equipment storage;

works of restoration;

bowsers septic tanks generators and standby generators;

ramps and temporary bridges;

temporary lighting;

works for the provision of apparatus including cabling water and electricity supply networks; and

archaeological works,

and in each case such other works, apparatus, plant and machinery of whatever nature as may be necessary or expedient for the purposes of or in connection with the relevant part of the authorised development.

The grid coordinates for that part of the authorised project which is seaward of MHWS are specified below and more particularly shown on the offshore Order limits and grid coordinates plan—

Table 1

| Point | Latitude | Longitude | Point | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- |
| 1 | 50° 48’ 0.6148 | -000° 32’ 46.9538 | 64 | 50° 39’ 22.8994 | -000° 11’ 0.3078 |
| 2 | 50° 48’ 0.3623 | -000° 32’ 46.6888 | 65 | 50° 39’ 22.9375 | -000° 11’ 0.1216 |
| 3 | 50° 47’ 58.5314 | -000° 32’ 44.7664 | 66 | 50° 39’ 22.9921 | -000° 10’ 59.8545 |
| 4 | 50° 47’ 55.2012 | -000° 32’ 41.2701 | 67 | 50° 39’ 23.0026 | -000° 10’ 59.8032 |
| 5 | 50° 47’ 54.8542 | -000° 32’ 41.2745 | 68 | 50° 39’ 23.0301 | -000° 10’ 59.6688 |
| 6 | 50° 47’ 47.1541 | -000° 32’ 41.3703 | 69 | 50° 39’ 23.0417 | -000° 10’ 59.6121 |
| 7 | 50° 47’ 24.0826 | -000° 32’ 41.6574 | 70 | 50° 39’ 23.0889 | -000° 10’ 59.3814 |
| 8 | 50° 46’ 40.6972 | -000° 32’ 42.1966 | 71 | 50° 36’ 40.5733 | -000° 10’ 7.2790 |
| 9 | 50° 45’ 42.5924 | -000° 32’ 42.9178 | 72 | 50° 35’ 56.8348 | -000° 12’ 29.9698 |
| 10 | 50° 44’ 50.3060 | -000° 32’ 43.5658 | 73 | 50° 35’ 46.1905 | -000° 13’ 4.6597 |
| 11 | 50° 43’ 27.9008 | -000° 32 44.5850 | 74 | 50° 35’ 26.4347 | -000° 14’ 9.0079 |
| 12 | 50° 43’ 4.3293 | -000° 32’ 44.8761 | 75 | 50° 35’ 44.1096 | -000° 20’ 10.6548 |
| 13 | 50° 42’ 28.3486 | -000° 32’ 45.3201 | 76 | 50° 36’ 25.6547 | -000° 21’ 2.8378 |
| 14 | 50° 42’ 25.1670 | -000° 32’ 45.4370 | 77 | 50° 36’ 27.9442 | -000° 21’ 5.7145 |
| 15 | 50° 42’ 22.6856 | -000° 32’ 42.9705 | 78 | 50° 36’ 33.7315 | -000° 21’ 12.9866 |
| 16 | 50° 41’ 28.6123 | -000° 31’ 57.7034 | 79 | 50° 36’ 36.0255 | -000° 21’ 15.9940 |
| 17 | 50° 41’ 12.8834 | -000° 31’ 44.5421 | 80 | 50° 36’ 42.3252 | -000° 21’ 24.2532 |
| 18 | 50° 39’ 52.3821 | -000° 30’ 37.2248 | 81 | 50° 36’ 50.0795 | -000° 21’ 34.4205 |
| 19 | 50° 41’ 11.3500 | -000° 21’ 55.8599 | 82 | 50° 37’ 0.5900 | -000° 21’ 48.2036 |
| 20 | 50° 39’ 53.9451 | -000° 21’ 30.9561 | 83 | 50° 37’ 2.6269 | -000° 21’ 50.8749 |
| 21 | 50° 38’ 43.2945 | -000° 21’ 8.2445 | 84 | 50° 37’ 26.1929 | -000° 22’ 21.7880 |
| 22 | 50° 38’ 11.9503 | -000° 20’ 58.1743 | 85 | 50° 37’ 27.1480 | -000° 22’ 23.0411 |
| 23 | 50° 37’ 42.3197 | -000° 20’ 48.6579 | 86 | 50° 36’ 38.8171 | -000° 27’ 24.4229 |
| 24 | 50° 37’ 32.8206 | -000° 20’ 45.6028 | 87 | 50° 36’ 16.5551 | -000° 29’ 46.1531 |
| 25 | 50° 37’ 32.7858 | -000° 20’ 45.5916 | 88 | 50° 36’ 12.3786 | -000° 30’ 12.7043 |
| 26 | 50° 37’ 32.7633 | -000° 20’ 45.5843 | 89 | 50° 35’ 33.2206 | -000° 34’ 21.0588 |
| 27 | 50° 37’ 22.7987 | -000° 20’ 42.3409 | 90 | 50° 35’ 29.7384 | -000° 34’ 43.2949 |
| 28 | 50° 38’ 31.7775 | -000° 16’ 26.9705 | 91 | 50° 35’ 11.5476 | -000° 36’ 40.5405 |
| 29 | 50° 38’ 13.3226 | -000° 16’ 17.0707 | 92 | 50° 35’ 8.6938 | -000° 36’ 58.9128 |
| 30 | 50° 38’ 41.3993 | -000° 14’ 22.9538 | 93 | 50° 35’ 8.5860 | -000° 37’ 35.2466 |
| 31 | 50° 39’ 19.2087 | -000° 11’ 18.3535 | 94 | 50° 35’ 15.1952 | -000° 37’ 36.0920 |
| 32 | 50° 39’ 19.3373 | -000° 11’ 17.7245 | 95 | 50° 35’ 17.9985 | -000° 37’ 36.4578 |
| 33 | 50° 39’ 19.5025 | -000° 11’ 16.9170 | 96 | 50° 35’ 18.0869 | -000° 37’ 36.4693 |
| 34 | 50° 39’ 19.6186 | -000° 11’ 16.3492 | 97 | 50° 35’ 25.0537 | -000° 37’ 37.3783 |
| 35 | 50° 39’ 19.6908 | -000° 11’ 15.9965 | 98 | 50° 35’ 31.1113 | -000° 37’ 38.1688 |
| 36 | 50° 39’ 19.7648 | -000° 11’ 15.6344 | 99 | 50° 35’ 52.0407 | -000° 37’ 40.9002 |
| 37 | 50° 39’ 20.4130 | -000° 11’ 12.4655 | 100 | 50° 36’ 22.3039 | -000° 37’ 44.8508 |
| 38 | 50° 39’ 20.4164 | -000° 11’ 12.4490 | 101 | 50° 36’ 24.7136 | -000° 37’ 45.1655 |
| 39 | 50° 39’ 20.6912 | -000° 11’ 11.1054 | 102 | 50° 36’ 36.3272 | -000° 37’ 46.6819 |
| 40 | 50° 39’ 20.6985 | -000° 11’ 11.0695 | 103 | 50° 37’ 8.9157 | -000° 37’ 51.3889 |
| 41 | 50° 39’ 20.7558 | -000° 11’ 10.7896 | 104 | 50° 37’ 14.4423 | -000° 37’ 52.1326 |
| 42 | 50° 39’ 20.7809 | -000° 11’ 10.6668 | 105 | 50° 37’ 26.2596 | -000° 37’ 53.7231 |
| 43 | 50° 39’ 20.8160 | -000° 11’ 10.4952 | 106 | 50° 37’ 33.2783 | -000° 37’ 54.6678 |
| 44 | 50° 39’ 20.8413 | -000° 11’ 10.3711 | 107 | 50° 37’ 47.0466 | -000° 37’ 56.5213 |
| 45 | 50° 39’ 20.8647 | -000° 11’ 10.2572 | 108 | 50° 37’ 52.4650 | -000° 37’ 57.2507 |
| 46 | 50° 39’ 20.8985 | -000° 11’ 10.0916 | 109 | 50° 37’ 55.7250 | -000° 37’ 57.5509 |
| 47 | 50° 39’ 20.9320 | -000° 11’ 9.9280 | 110 | 50° 39’ 1.7938 | -000° 36’ 8.7262 |
| 48 | 50° 39’ 20.9471 | -000° 11’ 9.8539 | 111 | 50° 39’ 24.3939 | -000° 33’ 40.8674 |
| 49 | 50° 39’ 20.9846 | -000° 11’ 9.6706 | 112 | 50° 40’ 16.3645 | -000° 34’ 24.3839 |
| 50 | 50° 39’ 21.0187 | -000° 11’ 9.5038 | 113 | 50° 41’ 39.4711 | -000° 35’ 34.0336 |
| 51 | 50° 39’ 21.0425 | -000° 11’ 9.3875 | 114 | 50° 42’ 42.0736 | -000° 36’ 26.5498 |
| 52 | 50° 39’ 21.0907 | -000° 11’ 9.1519 | 115 | 50° 42’ 54.3446 | -000° 36’ 23.8715 |
| 53 | 50° 39’ 21.1078 | -000° 11’ 9.0682 | 116 | 50° 43’ 29.5066 | -000° 36’ 16.1947 |
| 54 | 50° 39’ 21.1552 | -000° 11’ 8.8368 | 117 | 50° 44’ 36.7196 | -000° 36’ 1.5105 |
| 55 | 50° 39’ 21.1689 | -000° 11’ 8.7697 | 118 | 50° 45’ 10.4373 | -000° 35’ 54.1393 |
| 56 | 50° 39’ 21.1691 | -000° 11’ 8.7686 | 119 | 50° 46’ 18.6039 | -000° 35’ 39.2272 |
| 57 | 50° 39’ 21.2393 | -000° 11’ 8.4254 | 120 | 50° 46’ 50.5124 | -000° 35’ 32.2424 |
| 58 | 50° 39’ 21.2579 | -000° 11’ 8.3344 | 121 | 50° 47’ 32.2233 | -000° 35’ 23.1074 |
| 59 | 50° 39’ 21.2943 | -000° 11’ 8.1562 | 122 | 50° 47’ 34.7138 | -000° 35’ 22.5618 |
| 60 | 50° 39’ 21.3304 | -000° 11’ 7.9801 | 123 | 50° 47’ 36.2526 | -000° 35’ 22.2247 |
| 61 | 50° 39’ 21.3412 | -000° 11’ 7.9272 | 124 | 50° 47’ 37.5395 | -000° 35’ 21.9428 |
| 62 | 50° 39’ 22.1787 | -000° 11’ 3.8321 | 125 | 50° 47’ 37.7729 | -000° 35’ 21.8917 |
| 63 | 50° 39’ 22.8501 | -000° 11’ 0.5488 | 126 | 50° 47’ 38.9597 | -000° 35’ 21.6317 |

The grid coordinates for that part of the offshore Order limits where those parts of the authorised project comprising Work Nos. 1, 2(c) and 3(a) may be carried out as shown on the offshore works plans are specified below—

Table 2

| Point | Latitude | Longitude | Point | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- |
| 1 | 50° 39’ 1.7938 | -000° 36’ 8.7262 | 71 | 50° 39’ 27.3702 | -000° 30’ 22.4457 |
| 2 | 50° 39’ 24.3939 | -000° 33’ 40.8674 | 72 | 50° 39’ 27.5636 | -000° 30’ 20.9233 |
| 3 | 50° 39’ 38.5128 | -000° 32’ 8.3008 | 73 | 50° 39’ 27.7675 | -000° 30’ 19.3961 |
| 4 | 50° 39’ 38.0150 | -000° 32’ 7.9507 | 74 | 50° 39’ 27.9818 | -000° 30’ 17.8647 |
| 5 | 50° 39’ 37.5445 | -000° 32’ 7.5894 | 75 | 50° 39’ 28.2064 | -000° 30’ 16.3295 |
| 6 | 50° 39’ 37.0814 | -000° 32’ 7.2019 | 76 | 50° 39’ 28.4413 | -000° 30’ 14.7911 |
| 7 | 50° 39’ 36.6259 | -000° 32’ 6.7883 | 77 | 50° 39’ 28.6863 | -000° 30’ 13.2497 |
| 8 | 50° 39’ 36.1781 | -000° 32’ 6.3487 | 78 | 50° 39’ 28.9415 | -000° 30’ 11.7061 |
| 9 | 50° 39’ 35.7382 | -000° 32’ 5.8833 | 79 | 50° 39’ 29.2067 | -000° 30’ 10.1605 |
| 10 | 50° 39’ 35.3062 | -000° 32’ 5.3922 | 80 | 50° 39’ 29.4819 | -000° 30’ 8.6135 |
| 11 | 50° 39’ 34.8823 | -000° 32’ 4.8755 | 81 | 50° 39’ 29.7669 | -000° 30’ 7.0655 |
| 12 | 50° 39’ 34.4667 | -000° 32’ 4.3334 | 82 | 50° 39’ 30.0618 | -000° 30’ 5.5171 |
| 13 | 50° 39’ 34.0594 | -000° 32’ 3.7661 | 83 | 50° 39’ 30.3663 | -000° 30’ 3.9686 |
| 14 | 50° 39’ 33.6606 | -000° 32’ 3.1737 | 84 | 50° 39’ 30.6804 | -000° 30’ 2.4206 |
| 15 | 50° 39’33.2705 | -000° 32’ 2.5565 | 85 | 50° 39’ 31.0041 | -000° 30’ 0.8735 |
| 16 | 50° 39’ 32.8890 | -000° 32’ 1.9146 | 86 | 50° 39’ 31.3371 | -000° 29’ 59.3278 |
| 17 | 50° 39’ 32.5164 | -000° 32’ 1.2482 | 87 | 50° 39’ 31.6795 | -000° 29’ 57.7840 |
| 18 | 50°39’ 32.1528 | -000° 32’ 0.5575 | 88 | 50° 39’ 32.0310 | -000° 29’ 56.2425 |
| 19 | 50° 39’ 31.7982 | -000° 31’ 59.8427 | 89 | 50° 39’ 32.3917 | -000° 29’ 54.7039 |
| 20 | 50° 39’ 31.4528 | -000° 31’ 59.1040 | 90 | 50° 39’ 32.7613 | -000° 29’ 53.1685 |
| 21 | 50° 39’ 31.1167 | -000° 31’ 58.3418 | 91 | 50° 39’ 33.1398 | -000° 29’ 51.6369 |
| 22 | 50° 39’ 30.7900 | -000° 31’ 57.5561 | 92 | 50° 39’33.5271 | -000° 29’ 50.1095 |
| 23 | 50° 39’ 30.4727 | -000° 31’ 56.7473 | 93 | 50° 39’ 33.9230 | -000° 29’ 48.5867 |
| 24 | 50° 39’ 30.1651 | -000° 31’ 55.9155 | 94 | 50° 39’ 34.3275 | -000° 29’47.0691 |
| 25 | 50° 39’ 29.8671 | -000° 31’ 55.0611 | 95 | 50° 39’ 34.7403 | -000° 29’ 45.5571 |
| 26 | 50° 39’ 29.5788 | -000° 31’ 54.1843 | 96 | 50° 39’ 35.1614 | -000° 29’ 44.0512 |
| 27 | 50° 39’ 29.3005 | -000° 31’ 53.2853 | 97 | 50° 39’ 35.5907 | -000° 29’ 42.5518 |
| 28 | 50° 39’ 29.0320 | -000° 31’ 52.3645 | 98 | 50° 39’ 36.0279 | -000° 29’ 41. 0593 |
| 29 | 50° 39’ 28.7736 | -000° 31’ 51.4221 | 99 | 50° 39’ 36.4731 | -000° 29’ 39.5743 |
| 30 | 50° 39’ 28.5253 | -000° 31’ 50.4584 | 100 | 50° 39’ 36.9260 | -000° 29’ 38.0972 |
| 31 | 50° 39’28.2871 | -000° 31’ 49.4737 | 101 | 50° 39’ 37.3865 | -000° 29’ 36.6284 |
| 32 | 50° 39’ 28.0592 | -000° 31’ 48.4683 | 102 | 50° 39’ 37.8544 | -000° 29’ 35.1684 |
| 33 | 50° 39’ 27.8416 | -000° 31’ 47.4425 | 103 | 50° 39’ 38.3297 | -000° 29’ 33.7176 |
| 34 | 50° 39’ 27.6344 | -000° 31’ 46.3966 | 104 | 50° 40’ 22.3750 | -000° 27’ 19.7685 |
| 35 | 50° 39’ 27.4376 | -000° 31’ 45.3309 | 105 | 50° 40’ 53.6018 | -000° 23’ 53.4563 |
| 36 | 50° 39’ 27.2513 | -000° 31’ 44.2459 | 106 | 50° 37’ 23.6536 | -000° 22’ 44.8831 |
| 37 | 50° 39’ 27.0756 | -000° 31’ 43.1417 | 107 | 50° 36’ 38.8171 | -000° 27’ 24.4229 |
| 38 | 50° 39’ 26.9105 | -000° 31 42.0187 | 108 | 50° 36’ 16.5551 | -000° 29’ 46.1531 |
| 39 | 50° 39’ 26.7561 | -000° 31’ 40.8773 | 109 | 50° 36’ 12.3786 | -000° 30’ 12.7043 |
| 40 | 50° 39’ 26.6123 | -000° 31’ 39.7178 | 110 | 50° 35’ 33.2206 | -000° 34’ 21.0588 |
| 41 | 50° 39’ 26.4793 | -000° 31’ 38.5406 | 111 | 50° 35’ 29.7384 | -000° 34’ 43.2949 |
| 42 | 50° 39’ 26.3572 | -000° 31’ 37.3460 | 112 | 50° 35’ 11.5476 | -000° 36’ 40.5405 |
| 43 | 50° 39’ 26.2458 | -000° 31’ 36.1344 | 113 | 50° 35’ 8.6938 | -000° 36’ 58.9128 |
| 44 | 50° 39’ 26.1453 | -000° 31’ 34.9062 | 114 | 50° 35’ 8.5860 | -000° 37’ 35.2466 |
| 45 | 50° 39’ 26.0557 | -000° 31’ 33.6617 | 115 | 50° 35’ 15.1952 | -000° 37’ 36.0920 |
| 46 | 50° 39’ 25.9770 | -000° 31. 32.4013 | 116 | 50° 35’ 17.9985 | -000° 37’ 36.4578 |
| 47 | 50° 39’ 25.9093 | -000° 31’ 31.1254 | 117 | 50° 35’ 18.0869 | -000° 37’ 36.4693 |
| 48 | 50° 39’ 25.8526 | -000° 31’ 29.8344 | 118 | 50° 35’ 25.0537 | -000° 37’ 37.3783 |
| 49 | 50° 39’ 25.8068 | -000° 31’ 28.5286 | 119 | 50° 35’ 31.1113 | -000° 37’ 38.1688 |
| 50 | 50° 39’ 25.7720 | -000° 31’ 27.2086 | 120 | 50° 35’ 52.0407 | -000° 37’ 40.9002 |
| 51 | 50° 39’ 25.7482 | -000° 31’ 25.8746 | 121 | 50° 36’ 22.3039 | -000° 37’ 44.8508 |
| 52 | 50° 39’ 25.7355 | -000° 31’ 24.5271 | 122 | 50° 36’ 24.7136 | -000° 37’ 45.1655 |
| 53 | 50° 39’ 25.7337 | -000° 31’ 23.1665 | 123 | 50° 36’ 36.3272 | -000° 37’ 46.6819 |
| 54 | 50° 39’ 25.7265 | -000° 30’ 47.1665 | 124 | 50° 37’ 8.9157 | -000° 37’ 51.3889 |
| 55 | 50° 39’ 25.7358 | -000° 30’ 45.7932 | 125 | 50° 37’ 14.4423 | -000° 37. 52.1326 |
| 56 | 50° 39’ 25.7561 | -000° 30’ 44.4076 | 126 | 50° 37’ 26.2596 | -000° 37’ 53.7231 |
| 57 | 50° 39 25. 7874 | -000° 30’ 43.0102 | 127 | 50° 37’ 33.2783 | -000° 37’ 54.6678 |
| 58 | 50° 39’ 25.8297 | -000° 30’ 41.6014 | 128 | 50° 37’ 47.0466 | -000° 37’ 56.5213 |
| 59 | 50° 39’ 25.8830 | -000° 30’ 40.1815 | 129 | 50° 37’ 52.4650 | -000° 37’ 57.2507 |
| 60 | 50° 39’ 25.9472 | -000° 30’ 38.7511 | 130 | 50° 37’ 55.7250 | -000° 37’ 57.5509 |
| 61 | 50° 39’ 26.0225 | -000° 30’ 37.3106 | 131 | 50° 39’ 1.7938 | -000° 36’ 8.7262 |
| 62 | 50° 39’ 26.1086 | -000° 30’ 35.8603 | 132 | 50° 35’ 26.4347 | -000° 14’ 9.0079 |
| 63 | 50° 39’ 26.2056 | -000° 30’ 34.4008 | 133 | 50° 35’ 43.9510 | -000° 20’ 7.3806 |
| 64 | 50° 39’ 26.3136 | -000° 30’ 32.9325 | 134 | 50° 36’ 55.0393 | -000° 20’ 30.4937 |
| 65 | 50° 39’ 26.4323 | -000° 30’ 31.4559 | 135 | 50° 38’ 29.1370 | -000° 10’ 42.0738 |
| 66 | 50° 39’ 26.5619 | -000° 30’ 29.9713 | 136 | 50° 36’ 40.5733 | -000° 10’ 7.2790 |
| 67 | 50° 39’ 26.7022 | -000° 30’ 28.4793 | 137 | 50° 35’ 56.8348 | -000° 12’ 29.9698 |
| 68 | 50° 39’ 26.8532 | -000° 30’ 26.9803 | 138 | 50° 35’ 46.1905 | -000° 13’ 4.6597 |
| 69 | 50° 39’ 27.0149 | -000° 30’ 25.4747 | 139 | 50° 35’ 26.4347 | -000° 14’ 9.0079 |
| 70 | 50° 39’ 27.1873 | -000° 30’ 23.9630 |  |  |  |

The grid coordinates for that part of the offshore Order limits where the parts of the authorised project comprising Work No. 2(a), 2(b) and Work No. 3(b) may be carried out as shown on the offshore works plans are specified below—

Table 3

| Point | Latitude | Longitude | Point | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- |
| 1 | 50° 39’ 52.3821 | -000° 30’ 37.2248 | 48 | 50° 39’ 22.9375 | -000° 11’0.1216 |
| 2 | 50°41’ 11.3500 | -000° 21’55.8599 | 49 | 50° 39’22.9921 | -000° 10’59.8545 |
| 3 | 50° 39’ 53.9451 | -000° 21’30.9561 | 50 | 50° 39’ 23.0026 | -000° 10’ 59.8032 |
| 4 | 50° 38’ 43.2945 | -000° 21’ 8.2445 | 51 | 50° 39’ 23.0301 | -000° 10’ 59.6688 |
| 5 | 50° 38’ 11.9503 | -000° 20’ 58.1743 | 52 | 50° 39’ 23.0417 | -000° 10’ 59.6121 |
| 6 | 50° 37 42. 3197 | -000° 20’ 48.6579 | 53 | 50° 39’ 23.0889 | -000° 10’ 59.3814 |
| 7 | 50° 37’ 32.8206 | -000° 20’ 45.6028 | 54 | 50° 36’ 40.5733 | -000° 10’ 7.2790 |
| 8 | 50° 37’ 32.7858 | -000° 20’ 45.5916 | 55 | 50° 35’ 56.8348 | -000° 12’29.9698 |
| 9 | 50° 37’ 32.7633 | -000° 20’ 45.5843 | 56 | 50° 35’ 46.1905 | -000° 13’ 4.6597 |
| 10 | 50° 37’ 22.7987 | -000° 20’ 42.3409 | 57 | 50° 35’ 26.4347 | -000° 14’ 9.0079 |
| 11 | 50° 38’ 31.7775 | -000° 16’ 26.9705 | 58 | 50° 35’ 44.1096 | -000° 20’ 10.6548 |
| 12 | 50° 38’ 13.3226 | -000° 16’ 17.0707 | 59 | 50° 36’ 25.6547 | -000° 21’ 2.8378 |
| 13 | 50°38’ 41.3993 | -000° 14’ 22.9538 | 60 | 50° 36’ 27.9442 | -000° 21’ 5.7145 |
| 14 | 50° 39’ 19.2087 | -000° 11’ 18.3535 | 61 | 50° 36’ 33.7315 | -000° 21’ 12.9866 |
| 15 | 50° 39’ 19.3373 | -000° 11’ 17.7245 | 62 | 50° 36’ 36.0255 | -000° 21’ 15.9940 |
| 16 | 50° 39 19. 5025 | -000° 11’ 16.9170 | 63 | 50° 36 42. 3252 | -000° 21’ 24.2532 |
| 17 | 50° 39 19. 6186 | -000° 11’ 16.3492 | 64 | 50° 36’ 50.0795 | -000° 21’ 34.4205 |
| 18 | 50° 39 19. 6908 | -000° 11’ 15.9965 | 65 | 50° 37’ 0.5900 | -000° 21’ 48.2036 |
| 19 | 50° 39 19. 7648 | -000° 11’ 15.6344 | 66 | 50° 37’ 2.6269 | -000° 21’ 50.8749 |
| 20 | 50° 39’ 20.4130 | -000° 11’ 12.4655 | 67 | 50° 37’ 26.1929 | -000° 22’ 21.7880 |
| 21 | 50° 39’ 20.4164 | -000° 11’ 12.4490 | 68 | 50° 37’ 27.1480 | -000° 22’ 23.0411 |
| 22 | 50° 39’ 20.6912 | -000° 11’ 11.1054 | 69 | 50° 36’ 38.8171 | -000° 27’ 24.4229 |
| 23 | 50° 39’ 20.6985 | -000° 11’ 11.0695 | 70 | 50° 36’ 16.5551 | -000° 29’ 46.1531 |
| 24 | 50° 39’ 20.7558 | -000° 11’ 10.7896 | 71 | 50° 36’ 12.3786 | -000° 30’ 12.7043 |
| 25 | 50° 39’ 20.7809 | -000° 11’ 10.6668 | 72 | 50° 35’ 33.2206 | -000° 34’ 21.0588 |
| 26 | 50° 39’ 20.8160 | -000° 11’ 10.4952 | 73 | 50° 35’ 29.7384 | -000° 34’ 43.2949 |
| 27 | 50° 39’ 20.8413 | -000° 11’ 10.3711 | 74 | 50° 35’ 11.5476 | -000° 36’ 40.5405 |
| 28 | 50° 39’ 20.8647 | -000° 11’ 10.2572 | 75 | 50° 35’ 8.6938 | -000° 36’ 58.9128 |
| 29 | 50° 39’ 20.8985 | -000° 11’ 10.0916 | 76 | 50° 35’ 8.5860 | -000° 37’ 35.2466 |
| 30 | 50° 39’ 20.9320 | -000° 11’ 9.9280 | 77 | 50° 35’ 15.1952 | -000° 37’ 36.0920 |
| 31 | 50° 39’ 20.9471 | -000° 11’ 9.8539 | 78 | 50° 35’ 17.9985 | -000° 37’ 36.4578 |
| 32 | 50° 39’ 20.9846 | -000° 11’ 9.6706 | 79 | 50° 35’ 18.0869 | -000° 37’ 36.4693 |
| 33 | 50° 39’ 21.0187 | -000° 11’ 9.5038 | 80 | 50° 35’ 25.0537 | -000° 37’ 37.3783 |
| 34 | 50° 39’ 21.0425 | -000° 11’ 9.3875 | 81 | 50° 35’ 31.1113 | -000° 37’ 38.1688 |
| 35 | 50° 39’ 21.0907 | -000° 11’ 9.1519 | 82 | 50° 35’ 52.0407 | -000° 37’ 40.9002 |
| 36 | 50° 39’ 21.1078 | -000° 11’ 9.0682 | 83 | 50° 36’ 22.3039 | -000° 37’ 44.8508 |
| 37 | 50° 39’ 21.1552 | -000° 11’ 8.8368 | 84 | 50° 36’ 24.7136 | -000° 37’ 45.1655 |
| 38 | 50° 39’ 21.1689 | -000° 11’ 8.7697 | 85 | 50° 36’ 36.3272 | -000° 37’ 46.6819 |
| 39 | 50° 39’ 21.1691 | -000° 11’ 8.7686 | 86 | 50° 37’ 8.9157 | -000° 37’ 51.3889 |
| 40 | 50° 39’ 21.2393 | -000° 11’ 8.4254 | 87 | 50° 37’ 14.4423 | -000° 37’ 52.1326 |
| 41 | 50° 39’ 21.2579 | -000° 11’ 8.3344 | 88 | 50° 37’ 26.2596 | -000° 37’ 53.7231 |
| 42 | 50° 39’ 21.2943 | -000° 11’ 8.1562 | 89 | 50° 37’ 33.2783 | -000° 37’ 54.6678 |
| 43 | 50° 39’ 21.3304 | -000° 11’ 7.9801 | 90 | 50° 37’ 47.0466 | -000° 37’ 56.5213 |
| 44 | 50° 39’ 21.3412 | -000° 11’ 7.9272 | 91 | 50° 37’ 52.4650 | -000° 37’ 57.2507 |
| 45 | 50° 39’ 22.1787 | -000° 11’ 3.8321 | 92 | 50° 37’ 55.7250 | -000° 37’ 57.5509 |
| 46 | 50° 39’ 22.8501 | -000° 11’ 0.5488 | 93 | 50° 39’ 1.7938 | -000° 36’ 8.7262 |
| 47 | 50° 39’ 22.8994 | -000° 11’ 0.3078 | 94 | 50° 39’ 24.3939 | -000° 33’ 40.8674 |

The grid coordinates for that part of the offshore Order limits where that part of the authorised project comprising Work No. 4 may be carried out as shown on the offshore works plans are specified below—

Table 4

| Point | Latitude | Longitude | Point | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- |
| 1 | 50° 46’ 49.8007 | -000° 32’ 42.0835 | 97 | 50° 37’ 8.9157 | -000° 37’ 51.3889 |
| 2 | 50° 46’ 40.6972 | -000° 32’ 42.1966 | 98 | 50° 37’ 14.4423 | -000° 37’ 52.1326 |
| 3 | 50° 45’ 42.5924 | -000° 32’42.9178 | 99 | 50° 37’ 26.2596 | -000° 37’ 53.7231 |
| 4 | 50° 44’ 50.3060 | -000° 32 43.5658 | 100 | 50° 37’ 33.2783 | -000° 37’ 54.6678 |
| 5 | 50° 43’ 27.9008 | -000°32’ 44.5850 | 101 | 50° 37’ 47.0466 | -000° 37’ 56.5213 |
| 6 | 50° 43’ 4.3293 | -000° 32’ 44.8761 | 102 | 50° 37’ 52.4650 | -000° 37’ 57.2507 |
| 7 | 50° 42’ 28.3486 | -000° 32’ 45.3201 | 103 | 50° 37’ 55.7250 | -000° 37’ 57.5509 |
| 8 | 50° 42’ 25.1670 | -000° 32’ 45.4370 | 104 | 50° 39’ 1.7938 | -000° 36’ 8.7262 |
| 9 | 50° 42’ 22.6856 | -000° 32’ 42.9705 | 105 | 50° 39’ 24.3939 | -000° 33’ 40.8674 |
| 10 | 50° 41’ 28.6123 | -000° 31’ 57.7034 | 106 | 50° 40’ 16.3645 | -000° 34’ 24.3839 |
| 11 | 50° 41’ 12.8834 | -000° 31’ 44.5421 | 107 | 50° 41’ 39.4711 | -000° 35’ 34.0336 |
| 12 | 50° 39’ 52.3821 | -000° 30’ 37.2248 | 108 | 50° 42’ 42.0736 | -000° 36’ 26.5498 |
| 13 | 50° 41’ 11.3500 | -000° 21’ 55.8599 | 109 | 50° 42’ 54.3446 | -000° 36’ 23.8715 |
| 14 | 50° 39’ 53.9451 | -000° 21’ 30.9561 | 110 | 50° 43’ 29.5066 | -000° 36’ 16.1947 |
| 15 | 50° 38’ 43.2945 | -000° 21’ 8.2445 | 111 | 50° 44’ 36.7196 | -000° 36’ 1.5105 |
| 16 | 50° 38’ 11.9503 | -000° 20’ 58.1743 | 112 | 50° 45’ 10.4373 | -000° 35’ 54.1393 |
| 17 | 50° 37’ 42.3197 | -000° 20’ 48.6580 | 113 | 50° 46’ 18.6039 | -000° 35’ 39.2272 |
| 18 | 50° 37’ 32.8206 | -000° 20’ 45.6028 | 114 | 50° 46’ 31.3325 | -000° 35’ 36.4412 |
| 19 | 50° 37’ 32.7858 | -000° 20’ 45.5916 | 115 | 50° 46’ 30.8906 | -000° 35’ 31.1402 |
| 20 | 50° 37’ 32.7633 | -000° 20’ 45.5843 | 116 | 50° 46’ 30.6142 | -000° 35’ 24.4745 |
| 21 | 50° 37’ 22.7988 | -000° 20’ 42.3409 | 117 | 50° 46’ 30.6147 | -000° 35’ 17.7946 |
| 22 | 50° 38’ 31.7775 | -000° 16’ 26.9705 | 118 | 50° 46’ 30.8922 | -000° 35’ 11.1291 |
| 23 | 50° 38’ 13.3227 | -000° 16’ 17.0707 | 119 | 50° 46’ 31.4453 | -000° 35’ 4.5064 |
| 24 | 50° 38’ 41.3993 | -000° 14’ 22.9538 | 120 | 50° 46’ 32.2718 | -000° 34’ 57.9549 |
| 25 | 50° 39’ 19.2087 | -000° 11’ 18.3535 | 121 | 50° 46’ 33.3681 | -000° 34’ 51.5025 |
| 26 | 50° 39’ 19.3373 | -000° 11’ 17.7245 | 122 | 50° 46’ 34.7296 | -000° 34’ 45.1770 |
| 27 | 50° 39’ 19.5025 | -000° 11’ 16.9170 | 123 | 50° 46’ 35.4901 | -000° 34’ 42.2812 |
| 28 | 50° 39’ 19.6186 | -000° 11’ 16.3492 | 124 | 50° 46’ 35.5228 | -000° 34’ 42.0236 |
| 29 | 50° 39’ 19.6908 | -000° 11’ 15.9965 | 125 | 50° 46’ 35.5994 | -000° 34’ 41.2888 |
| 30 | 50° 39’ 19.7648 | -000° 11’ 15.6344 | 126 | 50° 46’ 35.6054 | -000° 34’ 41.2313 |
| 31 | 50° 39’ 20.4130 | -000° 11’ 12.4655 | 127 | 50° 46’ 35.9055 | -000° 34’ 38.5822 |
| 32 | 50° 39’ 20.4164 | -000° 11’ 12.4490 | 128 | 50° 46’ 36.0094 | -000° 34’ 37.7299 |
| 33 | 50°39’ 20.6912 | -000° 11’ 11.1054 | 129 | 50° 46’ 36.5355 | -000° 34’ 33.8272 |
| 34 | 50° 39’ 20.6985 | -000° 11’ 11.0695 | 130 | 50° 46’ 37.0004 | -000° 34’ 30.8806 |
| 35 | 50° 39’ 20.7558 | -000° 11’ 10.7896 | 131 | 50° 46’ 37.0010 | -000° 34’ 30.8770 |
| 36 | 50° 39’ 20.7809 | -000° 11’ 10.6668 | 132 | 50° 46’ 37.1267 | -000° 34’ 30.0528 |
| 37 | 50° 39’ 20.8160 | -000° 11’ 10.4952 | 133 | 50° 46’ 37.6810 | -000° 34’ 26.8347 |
| 38 | 50° 39’ 20.8414 | -000° 11’ 10.3711 | 134 | 50° 46’ 37.7138 | -000° 34’ 26.5748 |
| 39 | 50° 39’ 20.8647 | -000° 11’ 10.2572 | 135 | 50° 46’ 38.0846 | -000° 34’ 24.1951 |
| 40 | 50° 39’ 20.8985 | -000° 11’ 10.0916 | 136 | 50° 46’ 38.3829 | -000° 34’ 22.3753 |
| 41 | 50° 39’ 20.9320 | -000° 11’ 9.9280 | 137 | 50° 46’ 38.8484 | -000° 34’ 19.6871 |
| 42 | 50° 39’ 20.9471 | -000° 11’ 9.8540 | 138 | 50° 46’ 38.9607 | -000° 34’ 19.0720 |
| 43 | 50° 39’ 20.9846 | -000° 11’ 9.6706 | 139 | 50° 46’ 39.2202 | -000° 34’ 17.6871 |
| 44 | 50° 39’ 21.0187 | -000° 11’ 9.5038 | 140 | 50° 46’ 40.5811 | -000° 34’ 11.3611 |
| 45 | 50° 39’ 21.0425 | -000° 11’ 9.3875 | 141 | 50° 46’ 41.8814 | -000° 34’ 6.3216 |
| 46 | 50° 39’ 21.0907 | -000° 11’ 9.1519 | 142 | 50° 46’ 42.2559 | -000° 34’ 4.9738 |
| 47 | 50° 39’ 21.1078 | -000° 11’ 9.0682 | 143 | 50° 46’ 42.5759 | -000° 34’ 3.8412 |
| 48 | 50° 39’ 21.1552 | -000° 11’ 8.8368 | 144 | 50° 46’ 42.9023 | -000° 34’ 2.7225 |
| 49 | 50° 39’ 21.1689 | -000° 11’ 8.7697 | 145 | 50° 46’ 43.3239 | -000° 34’ 1.3002 |
| 50 | 50° 39’ 21.1691 | -000° 11’ 8.7686 | 146 | 50° 46’ 44.3721 | -000° 33’ 57.9966 |
| 51 | 50° 39’ 21.2393 | -000° 11’ 8.4254 | 147 | 50° 46’ 44.3810 | -000° 33’ 57.9636 |
| 52 | 50° 39’ 21.2579 | -000° 11’ 8.3345 | 148 | 50° 46’ 45.2637 | -000° 33’ 55.0152 |
| 53 | 50° 39’ 21.2944 | -000° 11’ 8.1562 | 149 | 50° 46’ 45.5808 | -000° 33’ 53.9984 |
| 54 | 50° 39’ 21.3304 | -000° 11’ 7.9801 | 150 | 50° 46’ 46.0305 | -000° 33’ 52.5844 |
| 55 | 50° 39’ 21.3412 | -000° 11’ 7.9272 | 151 | 50° 46’ 46.4507 | -000° 33’ 51.2883 |
| 56 | 50° 39’ 22.1787 | -000° 11’ 3.8321 | 152 | 50° 46’ 46.9907 | -000° 33’ 49.6586 |
| 57 | 50° 39’ 22.8501 | -000° 11’ 0.5488 | 153 | 50° 46’ 47.8546 | -000° 33’ 47.2974 |
| 58 | 50° 39’ 22.8994 | -000° 11’ 0.3078 | 154 | 50° 46’ 48.2437 | -000° 33’ 46.1079 |
| 59 | 50° 39’ 22.9375 | -000° 11’ 0.1216 | 155 | 50° 46’ 49.1345 | -000° 33’ 43.6730 |
| 60 | 50° 39’ 22.9921 | -000° 10’ 59.8545 | 156 | 50° 46’ 49.4912 | -000° 33’ 39.3906 |
| 61 | 50° 39’ 23.0026 | -000° 10’ 59.8033 | 157 | 50° 46’ 50.3164 | -000°33’ 32.8380 |
| 62 | 50° 39’ 23.0301 | -000° 10’ 59.6688 | 158 | 50° 46’ 51.1191 | -000° 33’ 27.9509 |
| 63 | 50° 39’ 23.0417 | -000° 10’ 59.6121 | 159 | 50° 46’ 51.3096 | -000° 33’ 26.8988 |
| 64 | 50° 39’ 23.0889 | -000° 10’ 59.3814 | 160 | 50° 46’ 51.3391 | -000° 33’ 26.6492 |
| 65 | 50° 36’ 40.5733 | -000° 10’ 7.2790 | 161 | 50° 46’ 51.5789 | -000° 33’ 24.7195 |
| 66 | 50° 35’ 56.8348 | -000° 12’ 29.9698 | 162 | 50° 46’ 51.6477 | -000° 33’ 24.1929 |
| 67 | 50° 35’ 46.1905 | -000° 13’ 4.6597 | 163 | 50° 46’ 51.7030 | -000°33’ 23.6624 |
| 68 | 50° 35’ 26.4347 | -000° 14’ 9.0079 | 164 | 50° 46’ 51.7067 | -000° 33’ 23.6272 |
| 69 | 50° 35’ 44.1096 | -000° 20’ 10.6548 | 165 | 50° 46’ 52.2817 | -000° 33’ 18.8380 |
| 70 | 50° 36’ 25.6547 | -000° 21’ 2.8378 | 166 | 50° 46’ 52.2855 | -000° 33’ 18.8103 |
| 71 | 50° 36’ 27.9442 | -000° 21’ 5.7145 | 167 | 50° 46’ 52.3480 | -000°33’ 16.5867 |
| 72 | 50°36’ 33.7315 | -000° 21’ 12.9867 | 168 | 50° 46’ 52.3819 | -000° 33’ 15.6368 |
| 73 | 50° 36’ 36.0255 | -000° 21’ 15.9940 | 169 | 50° 46’ 52.4500 | -000° 33’ 14.2166 |
| 74 | 50° 36’ 42.3253 | -000° 21’ 24.2532 | 170 | 50° 46’ 52.3827 | -000° 33’ 13.4068 |
| 75 | 50° 36’ 50.0795 | -000°21’ 34.4205 | 171 | 50° 46’ 52.2788 | -000° 33’ 12.3576 |
| 76 | 50° 37’ 0.5901 | -000° 21’ 48.2036 | 172 | 50° 46’ 52.1807 | -000° 33’ 11.5423 |
| 77 | 50° 37’ 2.6269 | -000° 21’ 50.8749 | 173 | 50° 46’ 52.1397 | -000° 33’ 11.1979 |
| 78 | 50° 37’ 26.1929 | -000° 22’ 21.7880 | 174 | 50° 46’ 52.0974 | -000° 33’ 10.8392 |
| 79 | 50° 37’ 27.1480 | -000° 22’ 23.0411 | 175 | 50° 46’ 51.8731 | -000° 33’ 8.8265 |
| 80 | 50° 36’ 38.8171 | -000° 27’ 24.4229 | 176 | 50° 46’ 51.3468 | -000° 33’ 2.6428 |
| 81 | 50° 36’ 16.5551 | -000° 29’ 46.1531 | 177 | 50° 46’ 51.3360 | -000° 33’ 2.4787 |
| 82 | 50° 36’ 12.3786 | -000° 30’ 12.7043 | 178 | 50°46’ 51.3359 | -000° 33’ 2.4775 |
| 83 | 50° 35’ 33.2206 | -000° 34’ 21.0588 | 179 | 50° 46’ 51.2831 | -000° 33’ 1.9360 |
| 84 | 50° 35’ 29.7384 | -000° 34’43.2949 | 180 | 50°46’ 51.0907 | -000° 32’ 59.8173 |
| 85 | 50° 35’ 11.5476 | -000° 36’ 40.5405 | 181 | 50° 46’ 50.8304 | -000° 32’ 56.7205 |
| 86 | 50° 35’ 8.6938 | -000° 36’ 58.9128 | 182 | 50° 46’ 50.6907 | -000° 32’ 54.9373 |
| 87 | 50° 35’ 8.5860 | -000°37’ 35.2466 | 183 | 50° 46’ 50.5692 | -000° 32’ 53.2671 |
| 88 | 50° 35’ 15.1952 | -000° 37’ 36.0920 | 184 | 50° 46’ 50.5517 | -000° 32’ 53.0243 |
| 89 | 50° 35’ 17.9985 | -000° 37’ 36.4578 | 185 | 50° 46’ 50.4801 | -000° 32’ 52.0184 |
| 90 | 50° 35’ 18.0869 | -000° 37’ 36.4693 | 186 | 50° 46’ 50.4366 | -000° 32’ 51.4437 |
| 91 | 50° 35’ 25.0537 | -000° 37’ 37.3783 | 187 | 50° 46’ 50.4028 | -000° 32’ 51.0278 |
| 92 | 50° 35’ 31.1113 | -000° 37’ 38.1688 | 188 | 50° 46’ 50.3262 | -000° 32’ 50.1580 |
| 93 | 50° 35’ 52.0407 | -000° 37’ 40.9002 | 189 | 50° 46’ 50.1892 | -000° 32’ 48.5032 |
| 94 | 50° 36’ 22.3039 | -000° 37’ 44.8508 | 190 | 50° 46’ 50.1151 | -000° 32’ 47.5480 |
| 95 | 50° 36’ 24.7136 | -000° 37’ 45.1655 | 191 | 50° 46’ 49.9511 | -000° 32’ 45.2090 |
| 96 | 50° 36’ 36.3272 | -000° 37’ 45.6819 |  |  |  |

The grid coordinates for that part of the offshore Order limits seaward of MLWS where the part of the authorised project comprising Work No. 5 may be carried out as shown on the offshore works plans are specified below—

Table 5

| Point | Latitude | Longitude | Point | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- |
| 1 | 50° 47’ 54.8542 | -000° 32 41.2745 | 44 | 50° 46’ 46.9907 | -000° 33’ 49.6586 |
| 2 | 50° 47’ 47.1541 | -000° 32’ 41.3703 | 45 | 50° 46’ 46.4507 | -000° 33’ 51.2883 |
| 3 | 50° 47’ 24.0826 | -000° 32’ 41.6574 | 46 | 50° 46’ 46.0305 | -000° 33’ 52.5844 |
| 4 | 50° 46’ 49.8007 | -000° 32’ 42.0835 | 47 | 50° 46’ 45.5808 | -000° 33’ 53.9984 |
| 5 | 50° 46’ 49.9511 | -000° 32’ 45.2090 | 48 | 50° 46’ 45.2637 | -000° 33’ 55.0152 |
| 6 | 50° 46’ 50.1151 | -000° 32’ 47.5480 | 49 | 50° 46’ 44.3810 | -000° 33’ 57.9636 |
| 7 | 50° 46’ 50.1892 | -000° 32’ 48.5032 | 50 | 50° 46’ 44.3721 | -000° 33’ 57.9966 |
| 8 | 50° 46’ 50.3262 | -000° 32’ 50.1580 | 51 | 50° 46’ 43.3239 | -000° 34’ 1.3002 |
| 9 | 50° 46’ 50.4028 | -000° 32’ 51.0278 | 52 | 50° 46’ 42.9023 | -000° 34’ 2.7225 |
| 10 | 50° 46’ 50.4366 | -000° 32’ 51.4437 | 53 | 50° 46’ 42.5759 | -000° 34’ 3.8412 |
| 11 | 50° 46’ 50.4801 | -000° 32’ 52.0184 | 54 | 50° 46’ 42.2559 | -000° 34’ 4.9738 |
| 12 | 50° 46’ 50.5517 | -000° 32’ 53.0243 | 55 | 50° 46’ 41.8814 | -000° 34’ 6.3216 |
| 13 | 50° 46’ 50.5692 | -000° 32’ 53.2671 | 56 | 50° 46’ 40.5811 | -000° 34’ 11.3611 |
| 14 | 50° 46’ 50.6907 | -000° 32’ 54.9373 | 57 | 50° 46’ 39.2202 | -000° 34’ 17.6871 |
| 15 | 50° 46’ 50.8304 | -000° 32’ 56.7205 | 58 | 50° 46’ 38.9607 | -000° 34’ 19.0720 |
| 16 | 50° 46’ 51.0907 | -000° 32’ 59.8173 | 59 | 50° 46’ 38.8484 | -000° 34’ 19.6871 |
| 17 | 50° 46’ 51.2831 | -000° 33’ 1.9360 | 60 | 50° 46’ 38.3829 | -000° 34’ 22.3753 |
| 18 | 50° 46’ 51.3359 | -000° 33’ 2.4775 | 61 | 50° 46’ 38.0846 | -000° 34’ 24.1951 |
| 19 | 50° 46’ 51.3360 | -000° 33’ 2.4787 | 62 | 50° 46’ 37.7138 | -000° 34’ 26.5748 |
| 20 | 50° 46’ 51.3468 | -000° 33’ 2.6428 | 63 | 50° 46’ 37.6810 | -000° 34’ 26.8347 |
| 21 | 50° 46’ 51.8731 | -000° 33’ 8.8265 | 64 | 50° 46’ 37.1267 | -000° 34’ 30.0528 |
| 22 | 50° 46’ 52.0974 | -000° 33’ 10.8392 | 65 | 50° 46’ 37.0010 | -000° 34’ 30.8770 |
| 23 | 50° 46’ 52.1397 | -000° 33’ 11.1979 | 66 | 50° 46’ 37.0004 | -000° 34’ 30.8806 |
| 24 | 50° 46’ 52.1807 | -000° 33’ 11.5423 | 67 | 50° 46’ 36.5355 | -000° 34’ 33.8272 |
| 25 | 50° 46’ 52.2788 | -000° 33’ 12.3576 | 68 | 50° 46’ 36.0094 | -000° 34’ 37.7299 |
| 26 | 50° 46’ 52.3827 | -000° 33’ 13.4068 | 69 | 50° 46’ 35.9055 | -000° 34’ 38.5822 |
| 27 | 50° 46’ 52.4500 | -000° 33’ 14.2166 | 70 | 50° 46’ 35.6054 | -000° 34’ 41.2313 |
| 28 | 50° 46’ 52.3819 | -000° 33’ 15.6368 | 71 | 50° 46’ 35.5994 | -000° 34’ 41.2888 |
| 29 | 50° 46’ 52.3480 | -000° 33’ 16.5867 | 72 | 50° 46’ 35.5228 | -000° 34’ 42.0236 |
| 30 | 50° 46’ 52.2855 | -000° 33’ 18.8103 | 73 | 50° 46’ 35.4901 | -000° 34’ 42.2812 |
| 31 | 50° 46’ 52.2817 | -000° 33’ 18.8380 | 74 | 50° 46’ 34.7296 | -000° 34’ 45.1770 |
| 32 | 50° 46’ 51.7067 | -000° 33’ 23.6272 | 75 | 50° 46’ 33.3681 | -000° 34’ 51.5025 |
| 33 | 50° 46’ 51.7030 | -000° 33’ 23.6624 | 76 | 50° 46’ 32.2718 | -000° 34’ 57.9549 |
| 34 | 50° 46’ 51.6477 | -000° 33’ 24.1929 | 77 | 50° 46’ 31.4453 | -000 35’ 4.5064 |
| 35 | 50° 46’ 51.5789 | -000° 33’ 24.7195 | 78 | 50° 46’ 30.8922 | -000° 35’ 11.1291 |
| 36 | 50° 46’ 51.3391 | -000° 33’ 26.6492 | 79 | 50° 46’ 30.6147 | -000° 35’ 17.7946 |
| 37 | 50° 46’ 51.3096 | -000° 33’ 26.8988 | 80 | 50° 46’ 30.6142 | -000° 35’ 24.4745 |
| 38 | 50° 46’ 51.1191 | -000° 33’ 27.9509 | 81 | 50° 46’ 30.8906 | -000° 35’ 31.1402 |
| 39 | 50° 46’ 50.3164 | -000° 33’ 32.8380 | 82 | 50° 46’ 31.3325 | -000° 35’ 36.4412 |
| 40 | 50° 46’ 49.4912 | -000° 33’ 39.3906 | 83 | 50° 46’ 50.5124 | -000° 35’ 32.2424 |
| 41 | 50° 46’ 49.1345 | -000° 33’ 43.6730 | 84 | 50° 47’ 32.2233 | -000° 35’ 23.1074 |
| 42 | 50° 46’ 48.2437 | -000° 33’ 46.1079 | 85 | 50° 47’ 32.8858 | -000° 35’ 22.9623 |
| 43 | 50° 46’ 47.8546 | -000° 33’ 47.2974 |  |  |  |

PART 2

ANCILLARY WORKS

Works within the Order limits which have been subject to an environmental impact assessment recorded in the environmental statement comprising—

temporary landing places, moorings or other means of accommodating vessels in the construction and/or maintenance of the authorised development;

marking buoys, beacons, fenders and other navigational warning or ship impact protection works; and

temporary works for the benefit or protection of land or structures affected by the authorised development.

PART 3

REQUIREMENTS

Time limits

— The authorised project must commence no later than the expiration of seven years beginning with the date this Order comes into force.

Detailed offshore design parameters

— The total number of wind turbine generators comprised in the authorised project must be no more than 90 having a total rotor swept area of no more than 4.45 square kilometres.

Subject to sub-paragraph (3), each wind turbine generator forming part of the authorised project must have—

a height of no more than 325 metres when measured from LAT to the tip of the vertical blade;

a rotor diameter of no more than 295 metres;

a distance of no less than 22 metres from MHWS to the lowest point of the rotating blade; or

a distance of no less than 830m from the nearest wind turbine generator in all directions,

and the authorised project will comprise turbines of a uniform height and rotor diameter.

The reference in sub-paragraph (2)(d) to the location of a wind turbine generator is a reference to the centre point of that wind turbine generator.

Wind turbine generator foundation structures forming part of the authorised development must be one of the following foundation options: monopile foundation, or multileg foundation.

Wind turbine generator—

multileg foundations employing pin piles forming part of the authorised project are to have—

no more than 4 legs per foundation;

no more than 4 pin piles per foundation; and

a pin pile diameter of no greater than 4.5 metres;

multileg foundations with suction bucket are to have—

no more than 4 legs per wind turbine generator;

a suction bucket diameter of no more than 15 metres; and

a suction bucket penetration of no more than 25 metres; and

monopile foundation forming part of the authorised project are to have a diameter no greater than 13.5 metres.

The total volume of scour protection comprising development associated with the wind turbine generators comprising Work No.1 must be no more than 1,215,000 cubic metres with a maximum footprint of 405,000 square metres.

— There must be no more than 3 offshore substations.

The dimensions of any offshore substation forming part of the authorised project must be no more than—

65 metres in height when measured from LAT and 115 metres in height with lightning protection and ancillary structures;

80 metres in length; and

50 metres in width.

There must be no offshore substation located within 500 metres of the array periphery.

Offshore substation foundation structures forming part of the authorised project must comprise either monopile foundations or multileg foundations.

Offshore substation installations with—

multileg foundation employing pin piles forming part of the authorised project are to have—

no more than 6 legs per foundation;

no more than 12 pin piles per foundation; and

a pin pile diameter of no greater than 4.5 metres; and

monopile foundation forming part of the authorised project are to have a diameter no greater than 13.5 metres.

The total volume of scour protection comprising development associated with the offshore substations comprising Work No. 3 must be no more than 65,700 cubic metres with a maximum footprint of 21,900 square metres.

— The total number of cable circuits must be no more than four.

The total length of the cable circuits comprising Work No. 2 must be no more than 250 kilometres.

The total volume of cable protection comprising development associated with Work No 2 must be no more than 175,000 cubic metres with a maximum footprint of 300,000 square metres.

The number of cable crossings comprising Work No. 2 must be no more than four unless otherwise agreed with the MMO.

The maximum area impacted by cable crossings must be no more than 10,000 square metres.

The total volume of cable protection for cable crossings must not exceed 20,000 cubic metres (pre- and post-lay).

The total length of the cable circuits comprising Work No. 3(b) must be no more than 40 kilometres.

The total volume of cable protection comprising development associated with Work No 3(b) must be no more than 110,500 cubic metres with a maximum footprint of 122,000 square metres.

The total length of cable circuits comprising Work Nos 3(b), 4 and 5 together must be no more than 170 kilometres.

The total volume of cable protection comprising development associated with Work Nos. 4 and 5 must be no more than 470,000 cubic metres with a maximum footprint of 517,000 square metres.

Cable parameters

— Cable circuits comprising Works Nos. 6 and 7 must be installed by means of horizontal directional drilling (HDD).

There must be no more than 4 HDD drills completed for Work Nos. 6 and 7.

All cable circuits and joint bays comprising Work Nos. 8, 9 and 19 must be installed in ducting underground and the land above reinstated to pre-construction ground level save for manholes and link box chambers and once constructed all joint boxes and link box chambers must be resilient to flooding.

Trenchless technology must be used to install the cable circuits in the locations identified in the crossings schedule (comprising part of the code of construction practice approved pursuant to requirement 22) for the purpose of passing under a relevant obstruction

There are to be no more than 66 joint bay locations each comprising up to four joint bays.

There are to be no more than 264 link boxes and no more than 264 fibre optic cable joint boxes.

Onshore substation parameters

— The total fenced area for the substation compound (excluding access) comprising Work No. 16 must be no more than 60,000 square metres.

The total area for the substation extension comprising Work No. 20 to be incorporated into the existing National Grid substation at Bolney must be no more than 6,300 square metres.

Site-specific plan for the detailed design approval of the onshore substation at Oakendene

— No stage which includes works comprising any part of Work No. 18, together with the parts of Work No. 17 associated with those works, accesses A61 and A64 as shown on sheet 33 of the access rights of way and streets plans at Kent Street, or establishment and use of either of the compounds comprising Work No. 10 shown on sheet 33 of the onshore works plans (excluding any onshore site preparation works) must commence until, only insofar as such works are included within or relate to works within that stage

The following details of the substation (Work No.16):

siting and layout;

scale and quantum of development and its uses;

existing and proposed finished ground levels;

landscaping;

access (Work No. 18); and

external appearance, form and materials for any buildings structures and other infrastructure including boundary treatment,

a single comprehensive traffic management and access plan covering:

the programming and routing of construction traffic along the A272 and into and out of the compounds comprising Work No. 10 shown on sheet 33 of the onshore works plans and Kent Street;

the safety of construction accesses off the A272 and Kent Street;

the safety of non-construction traffic, pedestrians, cyclists and equestrians using Kent Street;

the restriction of HGV traffic from unsuitable routes off the A272;

measures to minimise the volume of construction traffic passing through the Air Quality Management Area at Cowfold;

traffic management measures in respect of any previous stage approved under this requirement; and

details of vegetation removal, retention and replanting within Work Nos. 16 and 18 and along Kent Street at construction accesses A61 and A64 as shown on sheet 33 of the access rights of way and streets plans, which shall be consistent with the Outline Landscape and Ecology Management Plan

have been submitted to and approved in writing by the relevant planning authority following consultation with the highway authority and West Sussex Fire and Rescue Service and works covered by the details submitted under this sub-paragraph (1)must be carried out in accordance with the approved details.

Where details are approved pursuant to sub-paragraph (1) then details for

Construction traffic management;

Accesses A61 and A64 as shown on sheet 33 of the access rights of way and streets plans;

Vegetation removal, retention and replanting for the part of the stage for which approval has been secured under sub-paragraph 8(1);

shall not be required to be approved under requirements 12, 13, 15 24 as applicable.

Any details provided by the undertaker pursuant to sub-paragraph (1) must accord with the principles set out in the relevant part of the design and access statement including taking account of climate change allowances, relationship to and effect on heritage assets, must accord with the drainage arrangements approved pursuant to requirement 17, include details of any water harvesting and recycling measures and be within the Order limits.

The details submitted pursuant to sub-paragraph (3) must demonstrate how the operation of Work No. 16 will secure water neutrality.

To the extent comprised in Work No. 16—

there must be no more than 12 buildings;

the height of the main operational building and other infrastructure must be no more than 28.75 metres above ordnance datum;

the maximum main building length must be no more than 70 metres;

the maximum main building width must be no more than 20 metres;

lightning protection masts must be no more than a height of 34.25 metres above ordnance datum; and

the maximum height of any fire walls must be no more than 26.25 metres above ordnance datum.

For the purposes of paragraph (3) the term ‘water neutrality’ means that the use of water in the supply area before the development of Work No. 16 is the same or lower after the development is in place in order to meet the requirements of regulation 63 of the Conservation of Habitats and Species Regulations 2017.

For the purposes of paragraph (4) the term ‘building’ excludes electrical infrastructure installations.

Detailed design approval – extension to National Grid substation

— Works comprising Work No. 20 (excluding onshore site preparation works) must not commence until details of—

siting and layout;

scale and quantum of development and its uses;

existing and proposed finished ground levels;

landscaping;

access; and

external appearance, form and materials for any buildings structures and other infrastructure including boundary treatment,

of the extension to the National Grid substation at Bolney have been submitted to and approved in writing by the relevant planning authority in consultation with the highway authority and Work No. 20 must be carried out in accordance with the approved details.

Any details provided by the undertaker pursuant to sub-paragraph (1) must accord with the principles set out in the relevant part of the design and access statement unless otherwise agreed with the relevant planning authority, take account of climate change allowances, accord with the drainage arrangements approved pursuant to requirement 17, and be within the Order limits.

To the extent comprised in Work No. 20—

there must be no more than two buildings;

operational buildings must be no more than no more than 12 metres in height above finished ground level;

the maximum height of any associated infrastructure must be no more than 12 metres in height above finished ground level;

the maximum building length must be no more than 35 metres;

the maximum building width must be no more than 20 metres; and

the maximum height of any other infrastructure must be no more than 12 metres.

For the purposes of paragraph (3) “finished ground level” will be defined in accordance with the design and access statement.

Stages of onshore works

— No part of the authorised project within the Order limits landward of MLWS excluding onshore site preparation works is to commence until a written programme identifying the stages of those works has been submitted to and approved by the relevant planning authorities and to the extent that it relates to works seaward of mean high water springs comprising Work No. 6 following consultation with the MMO.

No onshore site preparation works are to commence until a written programme identifying the stages of those onshore site preparation works has been submitted to and approved by the relevant planning authorities and to the extent that it relates to works seaward of mean high water springs comprising Work No. 6 following consultation with the MMO.

The authorised project must be implemented in stages as approved and each reference to a stage in these requirements is a reference to a stage in the programme as so approved.

Offshore Decommissioning

No offshore works are to commence until a written decommissioning programme in compliance with any notice served upon the undertaker by the Secretary of State pursuant to section 105(2) of the 2004 Act() has been submitted to the Secretary of State for approval.

Provision of landscaping

— No stage of the authorised project within the onshore Order limits are to commence until for that stage a written landscape and ecology management plan and associated work programme (which accords with the relevant provisions of the outline landscape and ecology management plan and outline code of construction practice) has been submitted to and approved by the relevant planning authority following consultation with the statutory nature conservation body, Historic England (where relevant) and the Forestry Commission (where relevant).

Any landscape and ecology management plan submitted under sub-paragraph (1)—

may cover one or more stages of the onshore works; and

may provide for the phased provision of planting within a stage.

The landscape and ecology management plan for a stage which includes Work No. 16 and/or Work No. 20 (excluding for a stage comprising onshore site preparation works) is to include the relevant part of Work No. 17 and must accord with the relevant part of the design and access statement unless otherwise agreed by the relevant planning authority.

Each stage specific landscape and ecology management plan must be implemented as approved.

Implementation and maintenance of landscaping

— All landscape works must be carried out in accordance with the landscape and ecology management plan for the relevant stage approved under requirement 12 (provision of landscaping), and in accordance with the relevant recommendations of appropriate British Standards.

Any tree or shrub planted as part of an approved landscape and ecology management plan that, within a period of 10 years after completion of planting for the relevant stage, is removed by the undertaker, dies or becomes, in the opinion of the relevant planning authority, seriously damaged or diseased must be replaced in the first available planting season with a specimen of the same species and size as that originally planted unless otherwise approved in writing by the relevant planning authority.

Biodiversity net gain

— No stage of the authorised project within the onshore Order limits (excluding any onshore site preparation works) is to commence until—

a biodiversity net gain strategy for that stage which accords with the outline biodiversity net gain information comprising Appendix 22.15 of the Environmental Statement has been approved in writing by the relevant planning authority; and

at least 70% of the total number of biodiversity units as required for that stage of the development have been secured and where appropriate proof of purchase provided in accordance with the approved biodiversity net gain strategy and to the satisfaction of the relevant planning authority in consultation with the statutory nature conservation body.

The location for delivery of biodiversity units is to follow a prioritisation exercise, as described in Appendix 22.15 of the Environmental Statement, with priority given to areas inside or within close proximity to the proposed DCO limits.

The biodiversity net gain strategy for each relevant stage must be implemented as approved.

Any remaining shortfall in biodiversity units identified following detailed design will be secured prior to construction works being completed.

Any biodiversity net gain strategy under sub-paragraph (1) may cover one or more stages of the onshore works.

Highway accesses outside the South Downs National Park

— Construction of any—

new operational means of access to a highway to be used by vehicular traffic;

new temporary means of access to a highway to be used by vehicular traffic; or

alteration, or use of an existing means of access to a highway to be used by vehicular traffic,

outside the South Downs National Park must not commence until an access plan for the relevant access has been submitted to and approved by the highway authority for the relevant highway.

The access plans submitted pursuant to sub-paragraph (1) must include details of the siting, design, layout, visibility splays, access management measures and a maintenance programme relevant to the access it relates to and secure that all temporary and permanent accesses are provided to Department for Transport Design Manual for Roads and Bridges design standards or as otherwise agreed with the highway authority.

The highway accesses (including visibility splays) must be constructed and maintained for the period required in accordance with the details approved pursuant to sub-paragraph (1).

Highway accesses in the South Downs National Park

— Construction of any

new temporary means of access to a highway to be used by vehicular traffic; or

alteration, or us of an existing means of access to a highway to be used by vehicular traffic,

within the South Downs National Park is not to commence until written details of the siting, design, layout, visibility splays, access management (which shall be provided to Department for Transport Design Manual for Roads and Bridges design standards or as otherwise agreed with the highway authority) and a maintenance programme relevant to the access has been submitted to and approved by the highway authority for the relevant highway following consultation with South Downs National Park Authority.

The highway accesses within the South Downs National Park must be constructed and maintained for the period required in accordance with the details approved pursuant to sub-paragraph (1).

Operational drainage management

— No works to construct the onshore substation comprising Work No. 16 (excluding any onshore site preparation works) are to commence until an operational drainage plan including written details of the surface and (if any) foul water drainage system (including means of pollution control) and its management during the operational life of the authorised development, in accordance with the outline operational drainage plan has been submitted to and approved by the lead local flood authority following consultation with the relevant sewerage and drainage authorities and the Environment Agency.

The surface and foul water drainage system for Work No. 16 must be constructed and maintained in accordance with the approved details.

— No works to construct the extension of the National Grid substation comprising Work No. 20 (excluding any onshore site preparation works) are to commence until an operational drainage plan including written details of the surface and (if any) foul water drainage system (including means of pollution control) and its management during the operational life of the authorised development, in accordance with the outline operational drainage plan has been submitted to and approved by the lead local flood authority following consultation with the relevant sewerage and drainage authorities and the Environment Agency.

The surface and foul water drainage system for Work No. 20 must be constructed and maintained in accordance with the approved details.

Onshore Archaeology

— No stage of the authorised project within the onshore Order limits are to commence until a site-specific written scheme of archaeological investigation for that stage which must accord with the outline onshore written scheme of investigation has been submitted to and approved by the relevant planning authority following consultation with West Sussex County Council.

All archaeological investigations including those carried out as part of onshore site preparation works must be carried out in accordance with the relevant written scheme of archaeological investigation approved under sub-paragraph (1).

The archaeological site investigations and post investigation assessment must be completed in accordance with the programme set out in the written scheme of archaeological investigation and provision made for analysis, publication and dissemination of results and archive deposition.

Any written scheme or archaeological investigation submitted under sub-paragraph (1) may cover one or more stage of the onshore works.

In the event of the discovery of previously unknown archaeological remains within the onshore Order limits, their significance and suitability for preservation in situ must be assessed by field evaluation, in accordance with the outline onshore written scheme of investigation. Any archaeological remains which are demonstrably of national significance will be preserved in situ unless it is agreed by the relevant planning authority following consultation with West Sussex County Council, that either they are not suitable for preservation in situ or that preservation in situ cannot be achieved acceptably (including obtaining all necessary consents). Should archaeological remains be left in situ on any site, a site-specific archaeological management plan must be submitted to and approved in writing by the relevant planning authority following consultation with West Sussex County Council. Any further works, including removal and reinstatement, must be carried out in accordance with the approved site-specific archaeological management plan, unless otherwise approved by the relevant planning authority.

Public rights of way

— No stage of the authorised project within the onshore Order limits is to commence until a public rights of way management plan for the management of public rights of way (excluding the National Trail in the South Downs National Park) located within that stage and which accords with the outline public rights of way management plan has been submitted to and approved in writing by the highway authority following consultation with the relevant planning authority which is to include (where relevant for the stage) a programme for the temporary closure and re-opening of the public rights of way specified at Schedule 4 (public rights of way), save for the National Trail in the South Downs National Park, comprising—

a plan for the sequencing of construction of the onshore works;

the management of any alternative routes during the temporary closure; and

the re-opening of the public rights of way upon the cessation of that part of the authorised project requiring the temporary closure of those rights of way.

No stage of the authorised project within the onshore Order limits which includes the National Trail in the South Downs National Park is to commence until a public rights of way management plan for the relevant stage has been submitted to and approved in writing by the South Downs National Park Authority a programme for the temporary closure and re-opening of the National Trail within that stage, comprising—

a plan for the sequencing of construction of the onshore works;

the management of any alternative routes during the temporary closure; and

the re-opening of the National Trail upon the cessation of that part of the authorised development requiring the temporary closure of the National Trail.

Any public rights of way management plan submitted under sub-paragraph (1) may cover one or more stage of the onshore works.

The public rights of way management plan for each stage must be implemented as approved.

No stage of the onshore works are to commence unless the undertaker has provided to the highway authority and the South Downs National Park Authority a rights of way and access land communication management plan, which is to include—

proposals for informing the public of the start and duration of the connection works where public rights of way or the access land described in Schedule 6 (temporary suspension of public access to access land) are affected, including signage; and

details of the proposed diversions and temporary closures to minimise impacts on public rights of way during construction of the connection works,

and the details provided in accordance with this sub-paragraph (5) must be implemented during the construction of the relevant stages of the onshore works to which they relate.

Open access land

— No stage of the authorised project within which an area of access land is located is to commence until an open access land management plan for that area of access land has been submitted to and approved in writing by the highway authority following consultation with the relevant planning authority as part of the public rights of way management plan for the stage within which the open access land is located pursuant to requirement 20.

Any open access land management plan submitted under sub-paragraph (1) may cover one or more area of open access land.

Each open access land management plan submitted in accordance with sub-paragraph (1) must be implemented as approved.

Code of construction practice

— No stage of any works landward of MLWS is to commence until a detailed code of construction practice for the stage has been submitted to and approved by the relevant planning authority, following consultation with the Environment Agency, the statutory nature conservation body, the highway authority and the lead local flood authority and, to the extent that it relates to works seaward of mean high water springs comprising Work No. 6, the MMO.

Any construction code of practice submitted under sub-paragraph (1) may cover one or more stage of the works landward of MLWS.

All construction works landwards of MLWS must be undertaken in accordance with the relevant approved code of construction practice.

The code of construction practice must accord with the outline code of construction practice and include, as appropriate to the relevant stage—

arboricultural method statement and tree protection plan;

construction phase drainage plan;

materials management plan;

site waste management plan which accords with the outline site waste management plan;

soil management plan which accords with the outline soils management plan;

biodiversity management plan including invasive non-native species protocol;

noise and vibration management plan (construction) which accords with the outline noise and vibration management plan;

air quality management plan which accords with the outline air quality management plan;

emergency response plan;

pollution prevention plan and pollution incident response plan;

details for dealing with contaminated land and groundwater;

arrangements for temporary fencing or other means of secure enclosure of the construction sites and compounds during the carrying out of the connection works and their removal on their completion;

details for dealing with artificial light emissions (construction);

hours of construction working; and

water quality monitoring programme;

Onshore construction method statement

— No stage of the authorised project within the Order limits landward of MLWS is to commence until an onshore construction method statement for the construction of the relevant stage of the works which accords with the outline construction method statement has been submitted to and approved in writing by the relevant planning authority following consultation with the statutory nature conservation body and to the extent that it relates to works seaward of mean high water springs comprising Work No. 6, the MMO.

The onshore construction method statements referred to in paragraph (1) must, as appropriate to the relevant stage—

include details for the location, depth, method and process for horizontal directional drilling for the cable circuit installation works comprising Work Nos. 6 and 7 including restrictions on works above ground;

restrict access within ecologically sensitive sites including Climping Beach Site of Special Scientific Interest, Littlehampton Golf Course and Atherington Beach Local Wildlife Site, Sullington Hill Local Wildlife Site and ancient woodland at Michelgrove Park and Calcot Wood;

secure that the landfall construction compound comprising Work No. 8 do not exceed 120 metres x 100 metres;

secure that compounds associated with horizontal directional drilling do not exceed 50 metres x 75 metres;

secure appropriate burial depth for the onshore cable circuits comprising Work Nos. 9 and 19 within the relevant stage including under watercourses crossed by trenchless technology;

confirm the cable construction corridor location and its width through the relevant stage including that the width through treelines and areas of woodland is narrowed to no more than 30 metres;

include planned methods and processes for all crossings identified by the stage specific code of construction practice to be installed by trenchless technology; and

include a protocol for restoration and reinstatement of land used temporarily for construction including provisions to secure that the construction corridor through areas of woodland, across tree lines and important hedgerows (as defined pursuant to the Hedgerow Regulations 1997) are to be reinstated and which must accord with the stage specific landscape and ecological management plan approved pursuant to requirement 12 above.

Any construction method statement submitted under sub-paragraph (1) may cover one or more stage of the works landward of MLWS.

Each onshore construction method statement submitted in accordance with sub-paragraph (1) must be implemented as approved.

Construction traffic management plan

— No stage of the authorised project within the onshore Order limits landwards of MLWS is to commence until written details of

a construction traffic management plan (which accords with the outline construction traffic management plan); and

a construction workforce travel plan (which accords with the outline construction workforce travel plan),

for the stage have each been submitted to and approved by the highway authority following consultation with the relevant planning authority.

The construction traffic management plan must include, as a minimum—

a routeing plan to secure that heavy goods vehicles (HGVs) used during the construction period are to avoid settlements, the Air Quality Management Area in Cowfold and the A24 through Findon wherever possible;

management of junctions and crossings of the public highway; and

measures for laying cable circuits in streets by either single lane control or short road closure depending on the location.

Any construction traffic management plan submitted under sub-paragraph (1)(a) or construction workforce travel plan submitted under sub-paragraph (1)(b) may cover one or more stage of the onshore works.

The construction traffic management plan for each stage must be implemented as approved.

The onshore construction workforce travel plan for each stage must be implemented as approved.

Contamination risk

— No construction works excluding onshore site preparation works are to commence within the onshore Order limits in the localised areas identified within the preliminary risk assessment (desk study) included in chapter 25 of the environmental statement as being at risk from contamination until land contamination assessments have been undertaken for that localised area of the onshore Order limits and has been submitted to and approved by the relevant planning authority.

The land contamination assessments must be carried out in accordance with the Environment Agency’s Land Contamination Risk Management manual (“the LCRM”) and will include the following—

results of a site investigation scheme and quantitative risk assessment based on the findings of the preliminary risk assessment (desk study) to identify any site-specific assessment risks to receptors that may require a remediation strategy described in sub-paragraph (b);

where unacceptable risks are identified based on the results of the site investigation and risk assessment referred to in sub-paragraph (a), a remediation strategy giving full details of the remediation measures required and how they are to be undertaken;

a verification plan providing details of the data that will be collected in order to demonstrate that the works set out in the remediation strategy secured by sub-paragraph (b) are complete and have been effective; and

provision of a verification report in line with the verification plan referred to in sub-paragraph (c) prior to completion of the works.

If, during the course of construction, previously unidentified contamination is discovered, construction must cease on that localised area of land within the onshore Order limits (unless otherwise agreed in writing with the relevant planning authority) and the contamination reports to the relevant planning authority within 10 working days. Construction must not recommence on that localised area of land within the onshore order limits until a remediation strategy (including a verification plan) detailing how this contamination will be dealt with has been submitted to, and approved by, the relevant planning authority. The remediation strategy is to be developed in accordance with LCRM, which may include further site investigation and risk assessment, and must then be implemented as approved by the relevant planning authority.

Coastal erosion

— No works comprising Work Nos. 6 or 7 are to commence until a coastal erosion and future beach profile estimation assessment has been carried out and a scheme identifying any mitigation or adaptive management measures required to help minimise the vulnerability of this part of the Order land from future coastal erosion and tidal flooding (if required) has been submitted to and approved in writing by the Environment Agency.

Any mitigation or adaptive management measures identified as part of the scheme submitted pursuant to (1) above must be implemented as approved.

Operation phase maintenance

— The authorised development must not be brought into commercial use unless an operations and maintenance plan (OMP) for the onshore substation comprising Work No. 16 has been provided to the relevant planning authority.

The OMP provided pursuant to sub-paragraph (1) must include as a minimum—

a pollution prevention plan;

a pollution incident control plan; and

a risk assessment for major accidents and disasters.

— The authorised development must not be brought into commercial use unless an operations and maintenance plan (OMP) for the extension to the National Grid substation at Bolney comprising Work No. 20 has been provided to the relevant planning authority.

The OMP provided pursuant to sub-paragraph (1) must include as a minimum—

a pollution prevention plan;

a pollution incident control plan; and

a risk assessment for major accidents and disasters.

Control of noise during operational phase

— The onshore substation comprising Work No. 16 must not be commissioned unless an operational noise management plan (NMP) for the transmission substation has been submitted to and approved by the relevant planning authority.

The NMP submitted pursuant to sub-paragraph (1) must set out the particulars of—

the noise attenuation and mitigation measures to be taken to minimise noise resulting from the onshore substation, including any noise limits (subject to paragraph (3) below); and

a scheme for monitoring attenuation and mitigation measures provided under sub- paragraph (a) which must include—

the circumstances under which noise will be monitored;

limitations on noise levels at sensitive locations;

the locations at which noise will be monitored;

the method of noise measurement which must be in accord with the relevant British Standards or other agreed noise;

measurement methodology appropriate to the circumstances); and

a complaints procedure.

The NMP submitted pursuant to sub-paragraph (1) must ensure that the rating levels (noise emissions plus any character correction) from operation of the onshore substation comprising Work No. 16 will not exceed the noise levels set out in the design principles in the design and access statement.

The approved NMP must be implemented throughout the operational life of the authorised development.

Work No. 20 must not commence, excluding onshore site preparation works, until a plan has been submitted to the relevant planning authority which provides details of the equipment to be installed at the substation extension comprising Work No. 20 including:

its operational equipment that has the potential to generate noise emissions; and

characterisation of any sound and the expected frequency and duration of any such emissions to demonstrate that such emission will not have a discernible or measurable effect in the context of the existing noise environment at the National Grid substation at Bolney.

Control of artificial light emissions during operational phase

— Work No. 16 must not commence, excluding onshore site preparation works, until an operational light emissions management plan providing details of artificial light emissions during the operation of Work No. 16, including measures to minimise lighting pollution and the hours of lighting, in accordance with the design principles in the design and access statement, has been submitted to and approved by the relevant planning authority.

The approved lighting plan must be implemented throughout the operational life of the authorised development.

— Work No. 20 must not commence, excluding onshore site preparation works, until an operational light emissions management plan providing details of artificial light emissions during the operation of Work No. 20, including measures to minimise lighting pollution and the hours of lighting design principles in the design and access statement has been submitted to and approved by the relevant planning authority.

The approved lighting plan must be implemented throughout the operational life of the authorised development.

Operational Travel Plan

— Prior to the commencement of the operation of the authorised development an operation travel plan which must accord with the outline operational travel plan must be submitted to the relevant planning authority for approval following consultation with West Sussex County Council as highway authority.

The approved operation travel plan must be implemented throughout the operational life of the authorised development.

Skills and Employment Strategy

— No works comprising the authorised development are to commence, excluding onshore site preparation works, until a skills and employment strategy, in accordance with the outline skills and employment strategy, has been submitted to and approved by West Sussex County Council following consultation with the relevant planning authorities for all stages of the authorised development.

The skills and employment strategy must be implemented as approved.

Construction Communication Plan

— No stage of the authorised project landward of MLWS, excluding any onshore site preparation works, is to commence until a construction communication plan in accordance with the outline construction communication plan has been provided to and approved by the relevant planning authorities.

The construction communication plan must be implemented as approved throughout the construction of the authorised project within the onshore order limits.

Onshore decommissioning

— The undertaker must notify the relevant planning authority of the permanent cessation of commercial operation of the authorised development within 28 days following the date of permanent cessation.

Within six months of the cessation of commercial operation of the connection works an onshore decommissioning plan must be submitted to the relevant planning authority for approval unless otherwise agreed in writing by the relevant planning authority.

The approved decommissioning plan must be implemented unless otherwise agreed in writing by the relevant planning authority.

Requirement for written approval

Where the approval, agreement or confirmation of the Secretary of State, relevant planning authority, highway authority or another person is required under a requirement, that approval, agreement or confirmation must be given in writing.

Amendments to approved details

— With respect to any requirement which requires the authorised project to be carried out in accordance with the details approved by the relevant planning authority, highway authority or another person, the approved details must be carried out as approved unless an amendment or variation is previously agreed in writing by the relevant planning authority, highway authority or that other person in accordance with sub- paragraph (2).

Any amendments to or variations from the approved details must be in accordance with the principles and assessments set out in the environmental statement. Such agreement may only be given in relation to immaterial changes where it has been demonstrated to the satisfaction of the relevant determining authority or that other person that the subject matter of the agreement sought is unlikely to give rise to any materially worse environmental effects from those assessed in the environmental statement.

The approved details must be taken to include any amendments that may subsequently be approved in writing by the relevant determining authority or that other person.

Primary Surveillance Radar Mitigation

— No part of any wind turbine generator (excluding foundations) shall be erected as part of the authorised development until a primary radar mitigation scheme agreed in advance with the operator has been submitted to and approved in writing by the Secretary of State in order to mitigate the impact of the authorised development on the primary radar of the operator located at Pease Pottage and associated air traffic management operations.

No part of any wind turbine generator (excluding foundation) shall be erected until the approved primary radar mitigation scheme has been implemented and the authorised development shall thereafter be operated fully in accordance with the approved scheme.

For the purpose of paragraphs (1) and (2) above—

“operator” means NATS (En Route) plc, incorporated under the Companies Act (Company Number 4129273) whose registered office is 4000 Parkway, Whiteley, Fareham, Hampshire PO15 7FL or such other organisation licensed from time to time under sections 5 and 6 of the Transport Act 2000 to provide air traffic services to the relevant managed area (within the meaning of section 40 of the Transport Act); and

“primary radar mitigation scheme” or “scheme” means a detailed scheme agreed with the operator which sets out the measures to be taken to mitigate the impact of the development on the primary radar located at Pease Pottage and air traffic management operations of the Operator.

Instrument flight procedures

— Prior to commencement of the authorised development seaward of MHWS the undertaker must provide details of the number, specification (including lightning protection) and dimensions of each wind turbine generator and its location to the Airport Operator and provide confirmation of the same to the Secretary of State.

Within 21 days of providing the information specified in sub-paragraph (1) above, (or such other period as may be agreed in writing between the undertaker and the Secretary of State), the undertaker must, with any written confirmation from the Airport Operator received within that 21 day period, provide written confirmation to the Secretary of State that—

none of the turbines to be installed in accordance with the information submitted pursuant to sub-paragraph (1) above would have an impact on the instrument flight procedures of Shoreham Airport; or

a turbine or turbines to be installed in accordance with the details submitted pursuant to sub-paragraph (1) above or their turbine blades would have an impact on the instrument flight procedures of Shoreham Airport and that an IFP Scheme is required.

There shall be no commencement of the authorised development seaward of MHWS unless and until such time as—

the Secretary of State has confirmed and agreed in writing under paragraph 2(a); or

in the event that written notification under paragraph 2(b) is received, the Secretary of State has confirmed in writing that they are satisfied that the undertaker has put in place a binding undertaking to adequately compensate the Airport Operator for producing and securing the implementation of the IFP Scheme.

If an IFP Scheme is required, no wind turbine generator or turbine blade that has been identified as having an impact on instrument flight procedures may be erected or fitted unless and until such time as the Secretary of State receives written confirmation and evidence from the undertaker that—

an IFP Scheme has been approved by the Airport Operator; and

the Civil Aviation Authority has evidenced its approval to the Airport Operator of the IFP Scheme (if such approval is required).

For the purposes of this requirement—

“Airport Operator” means the operator of Shoreham Airport being Brighton City Airport Limited of The Terminal Building, Cecil Pashley Way, Shoreham-by-sea, West Sussex, BN43 5FF or any successor as holder of a licence under the Commission Regulation (EU) No. 139/2014 (or any successor regulation) from the Civil Aviation Authority to operate Shoreham Airport; and

“IFP Scheme” means a scheme to address the identified potential impact of certain wind turbine generators or turbine blades to be constructed on the instrument flight procedures of Shoreham Airport.

Vegetation retention and removal

— No stage of works landward of MLWS is to commence until a vegetation retention and removal plan (which accords with the outline vegetation retention and removal plan) has been submitted to and approved by the relevant planning authority.

Any vegetation retention and removal plan submitted under paragraph (1) may cover one or more stage of the onshore works.

The vegetation retention and removal plan for each stage must be implemented as approved.

Horsham District mitigation and enhancement

— The authorised project must not be commenced, save in respect of onshore site preparation works, until a Horsham District mitigation and enhancement scheme in accordance with the Horsham District mitigation and enhancement principles document has been submitted to and approved by Horsham District Council.

The Horsham District mitigation and enhancement scheme must set out appropriate measures to compensate for the impact of the authorised development on the landscape character in which the onshore substation is located.

The Horsham District mitigation and enhancement scheme must be implemented as approved.

In this requirement “Horsham District mitigation and enhancement principles document” means the document certified as such by the Secretary of State under article 51.

Public rights of way and landscape enhancement

— The authorised project must not be commenced, save in respect of onshore site preparation works until a public rights of way and landscape enhancement scheme in accordance with the public rights of way and landscape enhancement principles document has been submitted to and approved by West Sussex County Council.

The public rights of way and landscape enhancement scheme must set out appropriate measures to compensate for—

the impact of the loss of hedgerows and trees during the construction of the authorised development on the use and enjoyment of the public rights of way network in West Sussex; and

towards improvements to the public rights of way network affected by the construction of the authorised development within a 5km buffer zone of the onshore Order limits.

The public rights of way and landscape enhancement scheme must be implemented as approved.

In this requirement “public rights of way and landscape enhancement principles document” means the document certified as such by the Secretary of State under article 51.

National Park enhancement and furtherance

— The authorised project must not be commenced, save in respect of onshore site preparation works, until a National Park enhancement and furtherance scheme in accordance with the National Park enhancement and furtherance principles document has been submitted to and approved by South Downs National Park Authority.

The National Park enhancement and furtherance scheme must set out appropriate measures to compensate for the impact of the authorised development on the South Downs National Park and seek to further the statutory purposes of the South Downs National Park as set out in section 5 of the National Parks and Access to the Countryside Act 1949.

The National Park enhancement and furtherance scheme must be implemented as approved.

In this requirement “National Park enhancement and furtherance principles document” means the document certified as such by the Secretary of State under article 51.

European protected species licence

— No phase of the onshore works may commence until final pre-construction survey work has been carried out to establish whether a European protected species is present on any of the land affected, or likely to be affected, by that phase of the onshore works or in any of the trees to be lopped or felled as part of that phase of the onshore works.

Where a European protected species is shown to be present, the relevant phase of the onshore works must not commence until, after consultation with Natural England and the relevant planning authority, a scheme of protection and mitigation measures has been submitted to and approved by the relevant planning authority or a European protected species licence granted by Natural England.

The onshore works must be carried out in accordance with the approved scheme.

In this requirement “European protected species” has the same meaning as in regulations 42 and 46 of the 2017 Regulations.Site-specific plan for the detailed design approval of the temporary construction compounds at Washington and Climping

— Works to provide the temporary construction compounds comprising Work No. 10 must not commence until details of-

siting and layout;

scale and quantum of uses to be undertaken;

existing and proposed ground levels throughout the use of the compound;

access; and

infrastructure including boundary treatment for the temporary construction compounds;

have been submitted to and approved by the relevant planning authority following consultation with West Sussex County Council.

(2) Where details are approved pursuant to sub-paragraph (1) then details for access and egress points and visibility splays for the temporary construction compounds will not be required to be approved pursuant to requirements 15 or 24.

(3) The temporary construction compounds must be provided in accordance with the approved details.

Crossing Schedule

— No stage of the authorised development shall commence until a trenchless crossing plan showing the final locations and extent of each trenchless crossing in that stage and its compound has been submitted to and approved by the relevant planning authority.

The trenchless crossings in the relevant stages shall be undertaken in accordance with the approved details.

()	Section 105(2) was substituted by section 69(3) of the Energy Act 2008 (c. 32).