| Title: Clean Heat Market Mechanism IA No: DESNZ003(F)-24-NZBCH (2) RPC Reference No: RPC-DESNZ-5055(2) Lead department or agency: Department for Energy Security and Net Zero Other departments or agencies: None |  |  | Impact Assessment (IA). Date: 21/11/2024. Stage: Final. Source of intervention: Domestic. Type of measure: Secondary legislation. Contact for enquiries: heatmarketmechanism@energysecurity.gov.uk. |
| --- | --- | --- | --- |
| Summary: Intervention and Options |  |  | RPC Opinion: Green |  |
| Cost of Preferred (or more likely) Option |  |  |  |  |
| Total Net Present Social Value (2024 Prices, 2024 PV) £25m | Business Net Present Value (2024 Prices, 2024 PV) -£36m | Net Cost to Business Per Year (2024 Prices, 2024 PV) £9.5m |  | Business Impact Target Status Qualifying Provision |
| What is the problem under consideration? Why is government action or intervention necessary? Substantial acceleration in the rate of annual heat pump deployment is central to decarbonising heating and achieving net zero, since heat in buildings accounts for around 22% of annual UK greenhouse gas emissions. The deployment of electric heat pumps is also an integral part of measures to support greater energy independence and reduce dependence on imported oil and gas, since more than 40% of annual UK gas consumption is for heat in buildings. Along with measures such as insulation to improve the fabric efficiency of buildings, an important part of reducing energy demand is switching to more efficient heating appliances. As electric heat pumps can produce several units of heat for every unit of energy consumed, replacing a boiler with a heat pump is often the most impactful measure for reducing a building’s energy demand. To enable this level of deployment, the heat pump market and supply chains need to continue to develop substantially in order to provide a wide range of attractive choices for consumers, with compelling and simple consumer journeys, supported by a wide base of skilled advisors and installers. Heat pumps are currently largely unable to compete on lifetime cost with established fossil fuel and less efficient heating options, such as natural gas and oil boilers. This is partly due to the emerging nature of low-carbon heating, which means that it does not benefit from economies of scale or from mature supply chains to the same degree as conventional technologies. Additionally, the full societal costs of fossil fuel combustion are not reflected in their market prices, including for example impacts on health and climate change. In the absence of an effective policy framework, including regulatory policies such as the policy assessed here, for which powers have been granted in the Energy Act 2023, the heat pump market would not be expected to grow at the targeted rate. This would result in lower greenhouse gas emissions reductions from buildings than targeted in near-term carbon budgets. Without policies to develop the market and supply chain for heat pumps over the coming years to a sufficient capacity such as the policy assessed here, cost-effectively implementing further policy action necessary for reaching net zero emissions in the longer term may be at risk. What are the policy objectives of the action or intervention and the intended effects? The Clean Heat Market Mechanism (CHMM), as established by the Clean Heat Market Mechanism Regulations 2024, introduces a market obligation which establishes a platform for an industry-led transformation of the heating appliance market. As detailed in three consultations – in October 2021, March 2023 and March 2024 – the Clean Heat Market Mechanism creates a market incentive to grow the numbers of low-carbon heating appliances installed |  |  |  |  |
| each year, providing industry with a clear, long-term policy framework for investment and innovation. The scheme aims to: - Support the development of the UK heat pump market - Contribute to decarbonising heating in the UK and to meeting legally binding carbon budgets. This scheme sits within a wider policy framework aimed at expanding heat pump deployment and reducing greenhouse gas emissions from heating. It both complements and is complemented by other elements of that policy framework. The wider aims of government policy in this area are to: - Reduce the upfront costs of heat pumps and support households and building-owners with heat pump installations to kickstart the market; - Reduce the running costs of heat pumps, including relative to fossil fuel boilers; - Support an expansion of heat pump manufacturing in the UK; - Grow the numbers of skilled heat pump installers, and maintain high standards in the quality of heat pump installations; and - Support a range of innovations to broaden the appeal, efficiency and ease of adoption of heat pumps. Further detail on the government’s plans for supporting the decarbonisation of heating will be set out in due course as part of the Warm Homes Plan. A structural shift in the heating market such as this comes inevitably with high uncertainty, which is reflected in the estimates presented in this Impact Assessment. |  |  |  |  |
| What policy options have been considered, including any alternatives to regulation? Please justify preferred option (further details in Evidence Base) The policy options considered in this impact assessment are: - Option 0 (counterfactual): do nothing. In this scenario we assume there is no obligation on fossil fuel manufacturers to support deployment of low-carbon heating technologies. We assume in this scenario that heat pump deployment will be driven only by the current targeted spending and regulatory measures. - Option 1 (preferred option): introduce an obligation on the manufacturers of gas and oil boilers sold on the UK market to earn or acquire credits corresponding to qualifying installations of heat pumps in proportion to their relevant UK boiler sales. This policy is expected to continue to form part of a wider policy framework supporting heat decarbonisation; the combination of policies in this overall framework will have a bearing, for instance, on how policy costs are distributed across different groups. The principal alternatives to option 1 would be to pursue only subsidy-based measures and/or regulatory measures focused on consumers or building-owners without an accompanying market obligation. Such alternatives are less likely to reach the policy goals and would be likely to lead to higher overall social costs; they have been therefore disregarded from analysis. Option 1 has a positive Social Net Present Value (SNPV), meaning that the policy is expected to bring a net benefit to society, compared to the do-nothing scenario. Option 1 was selected as it is deemed to strike the best balance between driving heat pump uptake, achievability for businesses, and affordability. |  |  |  |  |
| Does implementation go beyond minimum EU requirements?; N/A. Is this measure likely to impact on international trade and investment?; Yes. Are any of these organisations in scope?; Micro No; Small No; Medium Yes; Large Yes. What is the CO2 equivalent change in greenhouse gas emissions? (Million tonnes CO2 equivalent); Traded: -0.1; Non-traded: 0.8. Will the policy be reviewed? It will be reviewed. If applicable, set review date: N/A |

I have read the Impact Assessment and I am satisfied that, given the available evidence, it represents a reasonable view of the likely costs, benefits and impact of the leading options.

| Signed by the responsible Minister: |  | Date: | 18/11/2024 |
| --- | --- | --- | --- |

Miatta Fahnbulleh

Summary: Analysis & Evidence – Policy Option 1

Description:  To introduce an obligation on the manufacturers of gas and oil boilers sold on the UK market to hold credits corresponding to qualifying installations of heat pumps in proportion to their relevant UK boiler sales.

FULL ECONOMIC ASSESSMENT

| Price Base Year | PV Base Year | Time Period Years: | Net Benefit (Present Value (PV)) (£m) |  |  |
| --- | --- | --- | --- | --- | --- |
| 2024 | 2024 | 22 years | Low: - | High: - | Best Estimate: £25m |

| COSTS (£m) | Total Transition |  | Average Annual | Total Cost (Present Value) |  |
| --- | --- | --- | --- | --- | --- |
| Low | - |  | - | - |  |
| High | - |  | - | - |  |
| Best Estimate | - |  |  | £195m |  |
| Description and scale of key monetised costs by ‘main affected groups’ The largest societal costs of the proposal are the additional capital costs associated with installing clean heating technologies, followed by traded carbon costs. These costs are estimated to be around £129m in additional capital costs and around £20m in traded carbon costs. Additionally, we have estimated administrative and familiarisation costs at around £29m and hidden costs associated with the installations of heat pumps at around another £6m. |  |  |  |  |  |
| Other key non-monetised costs by ‘main affected groups’ There will be some small costs to DESNZ and the administrator, which have not been monetised. |  |  |  |  |  |
| BENEFITS (£m) | Total Transition (Constant Price) Years |  | Average Annual (excl. Transition) (Constant Price) | Total Benefit (Present Value) |  |
| Low | - |  | - | - |  |
| High | - |  | - | - |  |
| Best Estimate | - |  | - | £220m |  |
| Description and scale of key monetised benefits by ‘main affected groups’ The largest monetised benefits are the carbon emissions savings in the non-traded sector (around £187m), followed by air quality improvements (around £34m). |  |  |  |  |  |
| Other key non-monetised benefits by ‘main affected groups’ Innovation benefits, reduced technology costs due to learning from wider deployment leading to future decarbonisation being more cost effective. Development of competitiveness in UK’s clean goods and services related to heat. Alignment with net zero strategy. Reduction of risks in other future policies. Growth in the market for low-carbon heating appliances and the businesses that produce, sell and install them, produce or operate ancillary goods (e.g., thermal storage, heat emitters) and services (e.g., smart energy management and flexibility services), etc. Policy framework stability, with market-wide application, enabling strategic confidence to invest in supply chains, training, etc. |  |  |  |  |  |
| Key assumptions/sensitivities/risks Discount rate – (3.5%) |  |  |  |  |  |
| The key assumptions, sensitivities and risks in the model are the deployment targets of other policies, the deployment targets for the market mechanism, and target carry-forward. This IA presents the uncertainty through sensitivity analysis in the Risks and Uncertainties section of this report. |  |  |  |  |  |

BUSINESS ASSESSMENT (Option 1)

| Direct impact on business, £m (Equivalent Annual, 2024 prices, 2024 PV) |  |  | Score for Business Impact Target, £m (qualifying provisions only, 2024 prices, 2024 PV) |
| --- | --- | --- | --- |
| Costs: £ | Benefits: | Net: -9.5mn |  |

# Executive Summary

The government has an ambitious package of policy measures to help the United Kingdom enhance its energy security, become less dependent on global gas markets, and reduce greenhouse gas emissions. One important way to enhance UK energy security and accelerate decarbonisation is to reduce the energy used in buildings. In addition to measures such as insulation to improve the fabric efficiency of buildings, an important part of reducing energy demand is switching to more efficient heating appliances. Electric heat pumps have a crucial role to play in this as they can produce several units of heat for every unit of energy consumed, meaning that replacing a boiler with a heat pump is often the most impactful measure for reducing a building’s energy demand. As the UK electricity generation mix continues to reduce in carbon intensity, heating using electric heat pumps will also continue to produce fewer greenhouse gas emissions.

As past consultations on the Clean Heat Market Mechanism set out, it is important that the heat pump market is supported to develop at pace over the coming years in order to make heat pumps a mainstream consumer solution alongside gas boilers, approximately 1.6 million of which are currently installed each year. Substantial growth in heat pump deployment, including in new-build properties, is strategically important for any energy transition pathway to net zero.

The Clean Heat Market Mechanism (CHMM) will, from 1 April 2025, place an obligation on the manufacturers of heating appliances to meet targets for the proportion of low-carbon heat pumps they sell each year, relative to fossil fuel boilers.

The Scheme Regulations, which this impact assessment accompanies, establish the target levels for the first year of the scheme; amending legislation will be required to establish any changes to the targets for years 2 to 4 of the scheme and adjust any other scheme parameters as may be necessary. If further legislation were not to be brought forward as planned, the Scheme Regulations as they are being laid would maintain the target level at the Year 1 level. Targets for Years 2-4 (and potentially beyond) will ultimately be set by the government, considering a range of factors including the feasible growth rate of the heat pump supply chain, the contribution of wider policy action to heat pump deployment, and the pace of market development necessary to meet strategic aims related to energy efficiency and emissions reductions. This impact assessment therefore assesses the impact of the Regulations being introduced, i.e. with the target set at 6% for year 1 of the scheme and without subsequent further target increases across the lifetime of the scheme.

The Regulations establish that a heat pump manufacturer will earn a credit (“a certificate”) for the installation of a qualifying domestic-scale hydronic heat pump notified via an appropriate certification scheme such as the Microgeneration Certification Scheme. To provide flexibility in how the obligation can be met, these credits are tradable. Providing further optionality in how the obligation may be met, certain installations of hybrid heat pump systems that involve both a heat pump and a fossil fuel boiler element also qualify, receiving 0.5 heat pump credits.

A range of other policies, such as the Boiler Upgrade Scheme, Social Housing Decarbonisation Fund and Energy Company Obligation, contribute to achieving the overall ambition for heat pump market growth. The Clean Heat Market Mechanism is designed to operate complementarily with this wider policy framework, with it both helping to shore up and maximise cost-effective delivery from other policies while the support to the heat pump sector from other policies helps ensure that the scheme’s targets are achievable and without disproportionate costs.

Contents

Executive Summary	6

Introduction & Background	8

Background	8

Rationale for intervention	8

Policy objective	10

Outline of policy options	10

Short list of options	11

Scheme design	12

Analytical approach	13

Heat Pump Deployment assumptions	14

Non-monetised costs and benefits	15

Modelling approach and results	16

Fuel Source Displaced	16

Impact appraisal	17

Risks and Uncertainties	19

Sensitivity Charts	21

Limitations	22

Distributional impact	22

Equality impact	22

Impact on Business	23

Familiarisation and admin costs	24

Obligation costs	25

Direct costs to businesses	27

Indirect costs to businesses	28

Competition and Trade Impact	29

Small and Micro Business Assessment (SaMBA)	30

Monitoring and Evaluation	31

Aims and Objectives	31

Monitoring metrics	32

Evaluation	34

Approach & Methodology	34

Evaluation delivery, outputs and timelines	34

Annex – full list of modelling assumptions and risks	36

# Introduction & Background

## Background

- In 2008, Parliament passed the Climate Change Act 2008 (‘the Act’) which sets a legally binding goal for reducing UK greenhouse gas (GHG) emissions by 2050. The original act, passed in 2008, committed the UK to an 80% reduction of greenhouse gas emissions by 2050, compared to 1990 levels. In 2019, the Climate Change Act 2008 (2050 Target Amendment) Order 2019 was passed which increased the UK’s commitment to reduce emissions by at least 100% below 1990 levels (i.e. Net Zero GHG emissions).

- The Act also requires the UK government to set ‘carbon budgets’, which are legally binding targets for UK emissions over five-year periods. The first five carbon budgets, running up to 2032, were set to achieve progress towards the previously set 80% reduction target by 2050. The Sixth Carbon Budget, set in June 2021, put into law a target to reduce greenhouse gas emissions to 965MtCO2e between 2033-2037 – meaning emissions will be approximately 77% lower (including international aviation and shipping) in 2035 than in 1990.

- To meet our carbon budgets, and to reach net zero emissions by 2050, the UK must decarbonise the vast majority of heat in buildings and most industrial processes. In 2022, the buildings sector was responsible for 22% of total UK net GHG emissions. This is a critical decade for action on the decarbonisation of heat and upgrading the energy efficiency of homes and other buildings.

- There are several possible pathways to full decarbonisation of heat by 2050 with a range of low-carbon technologies and systems that may have an important role to play, including a potential role for hydrogen. Heat pumps and heat networks are established technologies that will be the primary means for decarbonising heating over the next decade and play a key role in all 2050 scenarios.

- As consultations on the Clean Heat Market Mechanism set out, it is important that the heat pump market is supported to develop at pace over the coming years to make heat pumps a mainstream consumer solution alongside gas boilers. If widespread use of low-carbon gas heating, e.g. hydrogen, is ruled out as a viable strategic pathway for 2050, electrified heating based predominantly on heat pumps is likely to be the most widespread solution. This would require continued market growth to over one million annual heat pump installations by the early-2030s.

## Rationale for intervention

- The current market for heat pumps is relatively small: around 40,500 heat pumps were installed in pre-existing buildings in 2023, in comparison to around 1.6 million fossil fuel boiler installations (of which ~110,000 in new-build properties). Heat pumps are currently largely unable to compete on lifetime cost with established fossil fuel-based and less energy-efficient heating options, such as natural gas and oil boilers. This is partly due to the emerging nature of low-carbon heating, which means that it does not benefit from economies of scale or from mature supply chains to the same degree as conventional technologies.

- A key element of the rationale for this intervention is the market failure with respect to the uncaptured negative externalities of conventional heating technologies, which renders their market price too low compared to the price of heat pumps. The full societal costs of heating based on fossil fuel combustion should consider the impacts on health (related to the air quality impacts) and the emission of greenhouse gases, leading to climate change. The need to deliver advancements in the decarbonisation of heating requires more urgent government action to correct the effects of this market failure within the UK heating system.

- Likewise, the relative positive effect of heat pump deployment on air quality and emissions, and thus their lower societal cost, is not captured in their price. This is likely to result in under-investment in this technology, due to a lower expected payoff than would be provided by a market price reflecting the full range of social and private costs and benefits.

- As outlined in the 2023 Clean Heat Market Mechanism consultation document, there is already a set of existing and planned policies to stimulate heat pump deployment, including heat pump subsidies (e.g. the Boiler Upgrade Scheme) and building regulations (e.g. the proposed Future Homes Standard). However, this policy framework does not provide sufficient incentive and strategic confidence to relevant market actors to develop and promote products, offerings, and services that expand the consumer appeal and therefore uptake of heat pumps in retrofit settings. Without further intervention the heat pump market is unlikely to achieve the growth needed to establish the heat pump market quickly enough to meet our decarbonisation goals.

- The Clean Heat Market Mechanism (CHMM) is designed to work as part of this wider policy framework in order to provide further clarity and stimulus for investment and innovation throughout the manufacturing and installer supply chain and enable the heating industry to transform the consumer proposition of heat pumps in the UK.

- Ensuring that the running costs of a heat pump are as affordable as possible is another key enabler to mass deployment. The government is keen to explore all options for making electricity cheaper and wants to ensure that consumers are supported in order to make greener choices. As such, the government is considering the most effective and efficient way to bring down the cost of electricity and the running costs of low-carbon technologies.

- Some further reasons for intervention related to the above include:

  - Intervention in this market is needed to reduce the cost of decarbonising heat in buildings, as well as meeting legally-binding Carbon Budgets. Given the price effects of market failures set out above, support for the heat pump market, through a clear market growth trajectory underpinned by a market-wide obligation, is likely to improve investor certainty and generate growth and development of the supply chain.

  - Consumer research has shown that consumers are unfamiliar with heat pumps as an alternative to fossil fuel heating systems. This lack of consumer awareness reduces their ability to choose the heating appliance based on merit, and thus constraining the technology’s ability to compete in the market. One outcome from an intervention in the market would be to raise consumer awareness, addressing this market failure.

  - Additional research and development and economies of scale are also expected to follow a successful intervention in the heat pump market. This will result in spill-over benefits to society, which are not currently reflected in the price of low-carbon heating.

## Policy objective

- The Energy Act 2023 granted the Secretary of State the powers to introduce a market-based obligation for low-carbon heat in order to underpin an industry-led transformation of the heating appliance market. The introduction of this market obligation, the Clean Heat Market Mechanism, will create a firm market incentive to grow the numbers of heat pumps installed each year, providing industry with a clear, long-term policy framework for investment and innovation. This is expected to see the industry take a range of steps, directly and in partnership with other actors, to improve the consumer appeal and awareness of heat pumps in order to expand uptake.

- Heat pump deployment at significant scale – reaching the hundreds of thousands of annual retrofit installations over the coming years – is needed to make an electrification-led pathway to net zero a viable option at least-cost. Substantial further growth in annual installations will be needed by the early 2030s.

- The main aims of the policy are:

  - To support the development of the UK heat pump market to make heat pumps a mainstream consumer solution competing with gas boilers by the end of this Parliament. Working alongside other policies, the CHMM will help ensure the installation of at least 77,000 heat pumps a year in existing homes between 2024/25 and 2028/29.

  - To contribute to decarbonising heating in the UK and to meeting our legally-binding carbon budgets.

# Outline of policy options

- The policy options considered in this impact assessment are:

  - Option 0 (counterfactual): do nothing.

  - Option 1 (preferred option): introduce an obligation on the manufacturers of gas and oil boilers sold on the UK market to hold credits corresponding to qualifying installations of heat pumps in proportion to their relevant UK boiler sales.

- The CHMM Scheme Regulations establish the target level for the first year of the scheme; amending legislation will be required to establish any changes to the targets for years 2 to 4 of the scheme and adjust any other scheme parameters as may be necessary. If further legislation were not to be brought forward as planned, the target level would remain at the Year 1 level. Targets for Years 2-4 (and potentially beyond) will ultimately be set considering the feasible growth rate of the heat pump supply chain, the contribution of wider policy action to heat pump deployment, and the pace of market development necessary to meet strategic aims related to energy efficiency and emissions reductions. This impact assessment therefore assesses the impact of the Regulations as they are being introduced, i.e. with the target set at 6% for year 1 of the scheme and without subsequent further target increases across the lifetime of the scheme. If and when future proposals and/or legislation are brought forward to set higher targets for later scheme years, this will be accompanied by an updated Impact Assessment at that time.

- We have included a qualitative assessment of the potential impact on different businesses and consumers, as well as an indicative quantitative assessment of the possible scale of direct costs to business.

- The Clean Heat Market Mechanism is intended to work alongside targeted spending (e.g., the existing Boiler Upgrade Scheme) and regulatory measures (e.g., the planned Future Homes Standard) as part of an overall policy framework to support the development of the market in low-carbon heating. Future policies within this broader framework are currently at different stages of development. The principal overall policy alternative would therefore be to pursue either the same or further subsidy-based measures (e.g., heat pump grants) and/or the same or further building-owner regulatory measures alone on the demand side without an accompanying market obligation.

- Meeting the heat pump deployment ambition solely through subsidy-based measures would reduce the confidence of achieving the targeted deployment outcome, relative to our preferred option; this is because the risk would remain that insufficient consumer demand and grant uptake would be seen, likely because other non-cost barriers and inertia to uptake might not have been addressed. This option would also have lower incentives, relative to the preferred option, for industry to achieve downward pressure, through competitive market efficiencies, on the overall social cost. Over time, a large subsidy regime would risk locking in inefficiencies and higher prices. Further, this approach carries the usual risks associated with setting the correct subsidy level: grants too low would not generate the demand needed to meet the deployment ambition, while grants too high would over-compensate households reducing the benefit per pound of public spend. Therefore, this alternative is less likely to reach the policy goals and would likely lead to higher overall social costs.

- The same level of heat pump deployment could be achieved through regulatory measures alone, such as fossil fuel appliance installation bans for specific household segments. However, in this scenario there would be reduced incentives for an industry-led transformation of the market through competition, since the policy would provide industry with an obligated consumer base, potentially locking in expectations around current costs and prices. Further, this option would likely have significant negative distributional impacts with the targeted households forced to bear the full cost of heat decarbonisation. Therefore, this alternative is less likely to reach the policy goals and would lead to higher overall social costs or undermine the government’s goal of decarbonising the energy system in a fair and affordable way for all energy users.

- Finally, a combination of grants and installation bans would help to tackle only one of the market barriers (the higher installation cost of heat pumps compared to fossil fuel alternatives) but it would not provide any incentive for the market to experiment with efforts to address the other barriers preventing consumers from installing heat pumps. Therefore, under this option, deployment might meet the necessary rates, but it could do so for a higher cost to consumers and government, compared to our preferred option.

- For the reasons listed above, alternative options of meeting the deployment target without a market-based mechanism were discussed during earlier stages of analysis and further assessment was focused only on assessing the impact of different designs of the Clean Heat Market Mechanism.

## Short list of options

Option 0 (counterfactual): Do nothing

- In this Regulatory Impact Assessment, the quantified costs and benefits of an obligation on fossil fuel boiler manufacturers (option 1) is estimated against a counterfactual where no policy is introduced (option 0).

Option 1 (preferred option)

- The government is introducing, through secondary legislation under the primary powers granted by the Energy Act 2023, a new Clean Heat Market Mechanism (CHMM) from the financial year (FY) 2025/26, which will create a market incentive to grow the numbers of low-carbon heating appliances installed each year. This mechanism will work alongside a range of subsidy-based and regulatory policy approaches, targeted where most appropriate, to establish an overall policy framework capable of supporting a transformation of the market.

- As set out in the March 2023 consultation and accompanying consultation-stage impact assessment, the CHMM will create an obligation on the manufacturers of fossil fuel heating appliances (i.e., gas and oil boilers) to achieve the sale of a certain number of heat pumps, proportional to their fossil fuel boiler sales over a given period.

- In response, we would expect obligated parties to take a range of steps, both directly and in partnership with other market actors, to find and build consumer demand for heat pumps. In this way, we expect the policy to help create the conditions for rapid innovation across the market, for example, in consumer journeys and marketing, in products and product bundles, in service-based or consumer finance offerings, in the efficiency of or approach to surveys and installations, etc.

## Scheme design

- The CHMM will apply throughout the UK and apply to manufacturers of fossil fuel heating appliances (i.e., gas and oil boilers) with boiler sales above a minimum threshold.

- For the first year of the scheme, FY 2025/26, manufacturers of fossil fuel heating appliances will need to hold low-carbon heat pump credits corresponding to 6% of their relevant gas boiler sales in 2025/26 above 20,000 units and 6% of their relevant oil boiler sales above 1,000 units.

- The scheme will continue until FY 2028/29 with a target rate of 6%. If and when future proposals and/or legislation are brought forward to set higher targets beyond year 1, this will be accompanied by a consultation and updated Impact Assessment.

- Boiler sales to new-build developments and for export from the UK will be excluded. We therefore expect these targets to correspond to credits for qualifying heat pump installations totalling around 77,000 a year.

- For example, if a party sells 50,000 relevant gas boilers and 10,000 oil boilers between April 2025 and March 2026, its target for FY 2025/26 would be:

(50,000 – 20,000) * 6% + (10,000 – 1,000) * 6% = 2,340

- The sale on the UK market of fossil fuel boilers fired by natural gas, liquefied petroleum gas (LPG) or oil, up to 70kWth capacity, are relevant boiler sales under the CHMM and generate a corresponding obligation. Scheme credits will be issued for the installation of ‘air-to-water’, ‘ground-to-water’ or ‘water-to-water’ heat pumps up to 45kWth.

- High-temperature heat pumps will be treated the same as low-temperature heat pumps, but we will monitor market developments and consider whether any differentiation of treatment or weighting may be justified in later years of the scheme. For simplicity in this impact assessment, we have made the assumption that all installations under the obligation are low-temperature air-to-water heat pumps.

- Only boiler sales for the UK retrofit market will count towards the calculation of the obligation; boilers verifiably bound for export or for new buildings are excluded. Similarly, only heat pump installations in existing UK properties will count towards the obligation.

- Qualifying individual standalone heat pump installations will generate a single scheme credit for the manufacturer. Installations of qualifying hybrid heat pumps, systems that involve both a heat pump and a fossil fuel boiler element, will receive 0.5 heat pump credits. The lower weighting for hybrid heat pump installations compared with standalone heat pumps is intended to reflect: (1) the lower carbon reduction potential of these systems (given their reliance on fossil fuels); and (2) the lower energy demand reduction potential given their lower overall system efficiency, when compared to standalone heat pumps.

- In order to provide flexibility in how the obligation can be met, the scheme’s credits are tradable. Obligated parties are therefore able to meet their credit obligation, in part or in whole, through acquiring credits from other heat pump manufacturers. As there are many routes to compliance, in this IA we have only modelled the impact of the heat pump deployment, regardless of the approach obligated parties decide to take to comply with the regulation. In principle, a secondary market will not affect the total number of heat pumps installed so we have assumed it to have no impact on the overall social net present value. However, an illustrative analysis of the possible distributional impact to businesses of such a secondary market is presented in the Impact on Business section.

- Flexibility also comes from the fact that obligated scheme participants are able to carry forward a limited proportion of their target from one year to make up in the subsequent year, and from all manufacturers being allowed to carry forward some level of unsold or unused credits to the following year. We explore the possible impact of obligated parties carrying forward a share of their credit targets in the Risks and Uncertainties section.

- Credits for the first year of the obligation will be generated with respect to heat pumps installed between 1 April 2025 and 31 March 2026, between 1 April 2026 and 31 March 2027 for the second year of the obligation, and so on.

- Where obligated parties fail to meet their credit obligation, and where this shortfall exceeds the obligation that can be carried over into the subsequent year under the target carry-forward rules above, they will be required to make a payment in lieu of the remaining shortfall, at a rate of £500 per missing heat pump credit. This provides an incentive for companies to meet their targets through the sale and installation of heat pumps, or through the acquisition of heat pump credits, thereby ensuring the incentives align with the aim of the CHMM to support an expansion of the UK heat pump market. For these reasons, in the central scenario of this IA we have assumed full compliance of obligated parties. The Impact on Business section discusses the possible impact of payments-in-lieu on businesses.

# Analytical approach

- This section outlines the evidence base on which impacts of the scheme have been modelled and the overall analytical approach undertaken to assess the illustrative costs and benefits of the Clean Heat Market Mechanism. The impact assessment presents the evidence of the impacts of the proposals for households, the business sector and wider society. It follows the principle of the Green Book guidance in identifying the key direct costs and benefits for these groups. The changes are compared with a counterfactual scenario and then monetised using standard Green Book appraisal values. Net present values are derived by comparing the aggregate costs and benefits which are discounted by the social discount rate.

- This analytical approach has been scrutinised by the Regulatory Policy Committee (RPC) and has received a ‘Green’ rating. This is documented in the RPC Opinion Document reference: RPC-DESNZ-5055(2). The RPC’s opinion document contains summary figures based on an earlier, equivalent version of this impact assessment which used the same substantive approach and methodology. The RPC have agreed that their Green rating stands for this updated version of the impact assessment on the basis that it employs the same analytical approach to assessing the same overarching policy design.

- As described above, this impact assessment reflects the legislation as it will be introduced –manufacturers of fossil fuel heating appliances will need to hold low-carbon heat pump credits corresponding to 6% of their relevant boiler sales for each year of the policy, FY 2025/26 to FY 2028/29. If future proposals and/or legislation are brought forward to adjust targets beyond the first year of the scheme, this will be accompanied by a consultation and updated Impact Assessment.

- A cost-benefit approach is limited in assessing non-marginal change, such as the creation of markets and accelerated innovation, which are among the objectives of the proposal.  As such, the impact assessment is supplemented by a qualitative discussion of non-monetised costs and benefits which sets out the relevant evidence to wider strategic considerations.  Therefore, the calculated SNPVs are not intended to be viewed in isolation but should be assessed in combination with the strategic considerations.

## Heat Pump Deployment assumptions

- We anticipate that under Option 1 a 6% obligation target across a boiler market of 1,600,000 sales (the approximate size of the UK boiler market in 2023), taking account of minimum thresholds and other exclusions in the scheme, would lead to around 77,000 heat pump installations in the first year of the obligation.

- There are a wide range of UK Government schemes and policy action alongside the Clean Heat Market Mechanism which are designed to support the heat pump deployment, alongside action being taken by the Devolved Administrations. Any certified low-carbon heating retrofit installations performed in the relevant scheme year (e.g. FY 25/26) will count towards the obligation target set for that year.

- We expect the Clean Heat Market Mechanism to support or enable part of this deployment, for instance through contributing to market conditions in which more building-owners take decarbonisation action in response to building regulations earlier than in a counterfactual scenario, but it is difficult to produce a quantified estimate of this contribution. For simplicity, in the counterfactual scenario of this IA we have assumed that other policies would maintain the same level of low-carbon heating appliance deployment as in the absence of the CHMM. Therefore, in the policy scenario we have estimated only the impact of the additional deployment not primarily driven by (and therefore attributed to) other policies. This has been calculated as the difference between the target and the deployment that is already assumed will take place because of other policies (the counterfactual).

- This approach allows us to avoid any double counting of the impact of other policies. However, as we expect the market-based mechanism to be a partial enabler of the other policies, which tend to have a positive SNPV, the SNPV presented in this IA is likely to be a lower bound to the SNPV associated with the total retrofit deployment within heat pump targets.

- The ‘Clean Heat Market Mechanism additional deployment’ depends on the deployment levels of other policies. Estimating this level of deployment is challenging and subject to a high degree of uncertainty at this stage, especially since the installation of heat pumps is only one of the possible outcomes for certain policies in consideration. In this IA we have assumed that heat pump installations driven primarily by other policies will contribute to 61,000 credits in 2025/26. With the CHMM target for 2025/26 being 77,000, this would attribute the deployment of a further 16,000 HPs to the CHMM.

- In this IA, we are only accounting for CHMM additionality in 25/26 as, whilst future years’ budgets for other heat-pump-supporting schemes are yet to be finalised during the present multi-year Spending Review, we assume that they would collectively deliver significantly more deployment than the 77k CHMM target. Once the Spending Review is complete, and the Government has published further detail on its overall 'Warm Homes Plan', we will publish a further impact assessment focused on proposals for future CHMM scheme years

- Analysis has been conducted to estimate the costs and benefits associated with low-carbon heating technologies, relative to the counterfactual. The quantified costs and benefits contributing to the SNPV are:

- Additional upfront capital costs (Capex) - These are the total additional upfront costs of the purchase and installation of low-carbon heating technologies, compared to the purchase and installation costs of the counterfactual heating system. This includes additional ancillary costs such as new radiators for heat pumps and incorporates cost reductions assumed to materialise as the market scales up.

- Generation costs and benefits - The estimated value of the change in energy demand due to low-carbon heating technologies displacing counterfactual heating systems. This is also known as the long run variable cost (LRVC) impact.

- Carbon savings – The estimated value of the carbon abated in both the traded and non-traded sectors due to heat from low-carbon sources replacing heat from fossil fuels.

- Air quality impacts – The estimated value of the public health impacts of changes to emissions of nitrogen oxides and particulate matter.

- Maintenance – The difference between the annual costs to maintain the different heating systems. For simplicity, we have assumed these costs to be the same across all scenarios and we have therefore not included them in the tables below.

- Hidden costs – These include the time taken by householders to liaise with the installer, prepare the property for installation and any oversight.

- Admin and familiarisation costs – The costs associated with understanding the obligation and with compliance with the scheme once in force.

## Non-monetised costs and benefits

- There are several non-monetised costs and benefits that are not captured in the cost-benefit analysis, including:

- Supply chain development – by incentivising additional deployment of low-carbon heat technologies relative to the counterfactual, the scheme supports the development of low-carbon heat supply chains. This will provide a basis for the mass roll-out of low-carbon heating in the 2020s and subsequent decades, which will be needed to achieve the government’s target of net zero carbon emissions by 2050. It will also help create green jobs and create opportunities for UK manufacturers. If monetised, this would have a positive impact on the SNPV.

- Green jobs – the expansion of heat pump deployment is expected to support significant numbers of low-carbon and renewable energy (‘LCREE’) jobs. For instance the Heat Pump Association estimated that growth to 600,000 heat pump installations per year would support 30,000 LCREE jobs, predominantly in the installation and maintenance of heat pumps. This policy will contribute to securing these long-term low-carbon jobs.

- Innovation and cost reductions – DESNZ expects that supporting low-carbon heat deployment will reduce costs and possibly increase performance over time, as supply chains develop and barriers that customers currently face are reduced through technologies being deployed successfully. The cost reduction and performance improvement benefits from low-carbon heating technologies installed after the period in scope of the market mechanism are not quantified in this impact assessment. If monetised, they would have a positive impact on the SNPV.

- Health benefits – switching away from fossil fuels can lead to improved indoor air quality for occupants, improving their health. If monetised, this would have a positive impact on the SNPV.

- Consumer familiarity and perception towards renewable heat - the DESNZ Public Attitudes Tracker indicates that 51% of the public have little to no familiarity with air source heat pumps. However, customers who have installed renewable heating technologies have expressed high levels of satisfaction. Heat pumps would require consumers and businesses to operate their heating systems in an unfamiliar way compared to conventional heating systems. The installation of hundreds of thousands low-carbon heating appliances will improve the familiarity of the public with technologies essential to reach the net zero target. If monetised, this would have a positive impact on the SNPV.

# Modelling approach and results

- We have estimated the aggregate costs and benefits of the estimated additional clean heat installations over the years 2025/26 to 2028/29, appraised until 2045/46, when all the appliances installed are assumed to have reached the end of their lifetime.

## Fuel Source Displaced

- The design of the obligation leaves the obligated parties (and their consumer base) a high degree of freedom to choose in which buildings to install low-carbon heat appliances. The mix of building-types where heating systems primarily attributable to the CHMM will be installed is therefore uncertain, but has a significant impact on the estimated carbon savings in particular. For this analysis, we have assumed that the mix of heating types being replaced with heat pumps matches that seen with the Boiler Upgrade Scheme to date.

Table 1: Assumed Mix for Additional Market Mechanism Deployment of Heat Pumps

| Household Type | Deployment (%) | 25/26 Deployment Volumes |
| --- | --- | --- |
| On-gas-grid homes | 60.1% | 8k |
| Fossil fuel off-gas-grid homes | 28.5% | 5k |
| Electric off-gas-grid homes | 11.4% | 2k |

- This deployment is illustrative and assumes that BUS historical data represents the best indicator of future deployment and is maintained even as deployment levels grow. We have assumed that hybrid heat pump installations will constitute 20% of the additional on-gas-grid deployment from the market mechanism.

- While this cross-section of BUS installations by technology type forms our starting point, we have also imposed limits in the model which prevent retrofits for any incumbent technology type exceeding their annual turnover rate, as heat pump installations increase. Therefore, the mix of deployment can adjust over time with on grid retrofit often accounting for a larger share of the total by the end of the scheme. In this IA, we assume installations take place in the average household within each segment. We expect most of the heat pump deployment to take place in domestic buildings, but in practice it is possible that some installations may be in small non-domestic buildings. Given the thermal capacity limit for heat pump installations within the policy scope, we assume these non-domestic buildings have the same characteristics as domestic properties. Consequently, the proportion of non-domestic installations does not affect the quantifiable policy costs and benefits.

- Current evidence suggests that heat pumps are technically suitable for most buildings. DESNZ modelling suggests around 90% have sufficient energy efficiency and internal electrical connection capacity to accommodate a low temperature heat pump system. Other factors, such as space constraints, might reduce the proportion of buildings suitable for heat pumps in practice; however, it is unlikely that the obligated parties will target segments of the building stock where extensive new energy efficiency (e.g., insulation) measures are needed or where other factors could make the installation challenging. Therefore, in this IA we have not included the cost of any energy efficiency measures. We do however include the cost of in-home changes which we expect to be required in most buildings, such as hot water storage and larger radiators.

- We expect some households to deploy energy efficiency measures, which will reduce their heat demand, between now and the obligation period. However, low-carbon heating installations under the Clean Heat Market Mechanism will be on a voluntary basis so it is difficult to accurately predict the possible level of heat demand reduction in households benefitting from the mechanism. Therefore, as a modelling assumption in this IA we have assumed no impact on households’ heat demand because of energy efficiency improvements. This assumption carries the risk of overestimating both costs and benefits of the policy: a lower heat demand would lead to reduced heat pump installation costs and reduced carbon savings potential. Heat demand is assumed to change slightly from today’s level to account for long-term climate and energy trends.

- More detail on the modelling assumptions used and their source is given in the Annex.

## Impact appraisal

- Table 9 summarises the key social benefits and costs of the policy for a central scenario compared to the do-nothing counterfactual scenario. The Social Net Present Value (SNPV) represents the difference between these social costs and benefits i.e. the overall monetary value of the policy option relative to the do-nothing counterfactual. In the case of the CHMM, the SNPV represents the monetary value of the additional heat pump installations that are attributable to the policy option.

- Table 2: Monetised Costs – Option 1 (Central Scenario)

| £m, 2024 prices and PV year | Costs |
| --- | --- |
| LRVC | -12m |
| Capex | -129m |
| Traded Carbon Savings | -20m |
| Hidden costs | -6m |
| Admin + fam costs | -29m |
| Total Costs | -195m |

- The Capex value is negative as the cost of installing a heat pump is greater than that of common counterfactual technologies such as gas and oil boilers. The market-based and market-wide nature of the policy should help to keep overall costs as low as possible, with obligated parties competing to develop the heat pump market in the most efficient ways possible. A Eunomia report commissioned by DESNZ in 2022 suggests that deployment and R&D could bring down the total capital cost of heat pumps, including both appliance and installations costs, by around 20% in a scenario where deployment reaches 600,000 installations per year. Therefore, we have assumed that this level of cost reduction is achieved by 2028, with a linear reduction from 2024 prices. In practice, several market participants believe that higher cost reductions can be achieved significantly faster.

- Households that switch from gas boilers to heat pumps experience higher LRVC, despite the fact that heat pumps use less energy than counterfactual technologies (such as gas and oil boilers) to heat homes, due to the higher relative price of electricity. However, this is partly offset by homes, reliant on fossil fuel heating fuels, off the gas grid, where prices are more favourable to heat pumps. Overall, the LRVC impacts are a net cost. Maintenance costs are assumed to be the same in the policy scenarios and the counterfactual.

- Admin and familiarisation costs are discussed in more detail under the Impact on Business subheading.

- Table 3: Monetised Benefits – Option 1 (Central Scenario)

| £m, 2024 prices and PV year | Benefits |
| --- | --- |
| Non-Traded Carbon Savings | 187m |
| Air Quality | 34m |
| Total Benefits | 220m |

- In the central scenario, the majority of the monetised benefits arise from the non-traded carbon savings associated with the additional heat pump deployment attributable to the CHMM.

- The remainder of the monetised benefits are due to the positive air-quality impact from deploying heat pumps, relative to counterfactual technologies.

- Table 4: Carbon Savings - Option 1 (Central Scenario)

| Timeframe | Carbon Savings (MtCO2e) |
| --- | --- |
| Carbon Budget 4 Savings | 0.1 |
| Carbon Budget 5 Savings | 0.2 |
| Carbon Budget 6 Savings | 0.2 |
| Lifetime Savings (Non-Traded) | 0.8 |
| Lifetime Savings (Traded) | -0.1 |
| Lifetime Non-Traded CCE - £/tCO2 | 212 |

- The Non-Traded Carbon Cost Effectiveness (CCE) is the estimated average social cost of abating a tonne of carbon dioxide equivalent. Whilst the CCE is a value-for money measure, it does not all wider benefits, such as air quality impacts, associated with the policy option and so should be used with caution.

- Table 5: SNPVs – Option 1 (Central Scenario)

| £m, 2024 prices and PV year |  |
| --- | --- |
| Non-Traded Carbon Savings | 187m |
| Air Quality | 34m |
| LRVC | -12m |
| Capex | -129m |
| Traded Carbon Savings | -20m |
| Hidden costs | -6m |
| Admin + fam costs | -29m |
| Total Costs | -295m |
| Total Benefits | 220m |
| SNPV | 25m |

- The social benefits associated with the policy option in the central scenario more than offset the corresponding social costs, as indicated by the positive SNPV value.

# Risks and Uncertainties

- In this section, sensitivity analysis is performed against our central case, the impact of the scheme regulations. The quantified impacts laid out in the previous section are sensitive to changes in the underlying assumptions. Therefore, in this section we have tested lower and upper bounds on key input assumptions, which are discussed below. A complete list of modelling assumptions and risks is provided in the annex.

S1: Carbon prices

- Low and high carbon value projections in the Green Book guidance are used for this sensitivity test.

S2: Energy prices

- Low and high long run variable costs in the Green Book guidance are used to test the sensitivity on energy prices, which are expected to be highly uncertain.

S3: Efficiency of heating system

- The efficiency of a low-carbon heating system has an impact on fuel consumption and running costs.  This is expected to vary with weather conditions, the nature of the building stock, and the level of innovation. The low and high end of the assumption range is tested here (S3). This sensitivity test is also intended to reflect uncertainty with regards to future improvements of clean heat system performance.

S4: Future capital cost reduction for heat pumps

- A Eunomia report commissioned by DESNZ in 2022 shows that deployment and R&D could bring down the total capital cost of heat pumps, including both appliance and installations costs, by around 20% in a mass market scenario. In the central scenario we assumed this reduction to take place by 2028, with a linear reduction trajectory from 2024 prices. In the lower/upper bounds of this sensitivity analysis (S3), we explore scenarios with linear cost reduction trajectories leading to 10%/50% reductions by 2030.

S5: Deployment assumptions – hybrid deployment

- Compared to standalone heat pumps, hybrids imply lower emissions savings as fossil fuels are used to meet part of the heat demand. The level of emissions savings for each fossil fuel hybrid installation is proportionate to the level of heat demand met by the heat pump component. In the central scenario we have assumed that 20% of the heat pumps installed in households on the gas grid are hybrids; in this sensitivity (S5), we explore the impact of a higher (50%) and lower (0%) proportion. Because of the planned credit weighting (0.5 credits for a hybrid installation vs. 1 credit for a standalone heat pump), varying the proportion of hybrid deployment has an impact on the total number of heat pumps (hybrid and standalone) deployed.

S6: Target carry-forward

In this scenario we have assumed that obligated parties carry forward a proportion of their target from one year to the next, in line with the regulation.

Table 6: Sensitivity Results for Years 1 And 2 Of the Scheme

| Scenario / | sNPV (£mn) | CB6 Savings (N/T) |
| --- | --- | --- |
| Impact Assessment of the Scheme Regulations – Central Case | 25m | 0.2 |
| S1: Carbon Values - Lower | -58m | 0.2 |
| S1: Carbon Values - Higher | 109m | 0.2 |
| S2: Energy Prices - Lower | -7m | 0.2 |
| S2: Energy Prices - Higher | 102m | 0.2 |
| S3: Efficiency of Heating System - Lower | 8m | 0.2 |
| S3: Efficiency of Heating System - Higher | 41m | 0.2 |
| S4: Future Capital Cost Reduction for Heat Pumps – Lower | 14m | 0.2 |
| S4: Future Capital Cost Reduction for Heat Pumps – Higher | 39m | 0.2 |
| S5: Penetration of Hybrid Heat Pumps Within On Gas Grid Homes – Lower | 26m | 0.2 |
| S5: Penetration of Hybrid Heat Pumps Within On Gas Grid Homes – Higher | 23m | 0.2 |
| S6: Carryover Used - High | -29m | 0.0 |

## Sensitivity Charts

Figure 1 – SNPV Sensitivity Analysis of the Scheme Regulations

## Limitations

- Despite the sensitivity analysis conducted to flex assumptions; we are also aware of some of the limitations of the modelling. For example, we haven’t been able to consider the opportunity cost of capital due to the uncertainty on the scale of private funds used. Supplementary guidance to the Green Book on valuing energy use and greenhouse gas emissions suggests that when capital is tied up in a specific project, alternative profitable use of such capital is ruled out and there is a foregone social benefit.

- While the SNPV value presented is positive, there are the following non-monetised risks and uncertainties to consider:

  - Risk of non-compliance. HMT Green Book guidance requires that central scenarios assume compliance with regulation. However, should obligated parties decide to make payment in lieu of heat pump sales, this will reduce the growth of the heat pump market and corresponding benefits. The rate of the payment in lieu has been set to mitigate this risk. This is discussed further in the impacts on business section below.

  - Impact on cost reductions. Possible supply chain constraints might decrease the ability of the policy to reduce heat pump costs over time. The government is taking steps to mitigate this risk including a £5m Heat Training Grant to support trainees in England taking training relevant to heat pumps and heat networks. This is expected to support 10,000 training opportunities up to 2025, including at least 6,000 for heat pumps.

# Distributional impact

- The government is committed to keeping the overall policy framework for net zero under review, including how costs associated with the transition to low-carbon heating are distributed across different consumer groups. At the heart of this are efforts to help ensure that low-income and fuel-poor households are not disproportionately affected and that there is support where it is needed to make sure the transition is accessible and affordable across society.

- As heat pump uptake under this scheme is on a voluntary basis, we do not expect any impact on fuel poverty. Households switching from direct electric heating to heat pumps will see a significant reduction in their energy demand because of the higher efficiency of heat pumps, with a beneficial impact on energy bills. Reducing the running costs of heat pumps, including relative to fossil fuel boilers, is an aim of wider government policy. The government is keen to ensure that consumers can make greener choices  and is considering the best and most efficient way to bring down running costs of low-carbon alternatives such as heat pumps.

# Equality impact

- Under the Public Sector Equality Duty, the government must have due regard to the potential impact of the market-based mechanism on people with protected characteristics as set out in s.149 of the Equality Act 2010 (age, disability, gender reassignment, marriage or civil partnership, pregnancy and maternity, race, religion or belief, sex, sexual orientation). This requires DESNZ to pay due regard to the need to:

  - eliminate unlawful discrimination, harassment and victimisation and other conduct prohibited by the Act;

  - advance equality of opportunity between people who share a protected characteristic and those who do not;

  - foster good relations between people who share a protected characteristic and those who do not.

- The main groups that will be affected by the policy are:

- Households – who will decide to install the heat pumps deployed by the obligated parties

- Installers of heat pumps – who will be contracted for installations o low carbon heat technologies

- Manufacturers – who will need to meet the obligation.

- Equalities Impact Assessments have been conducted throughout the scheme’s development; and a summary is provided here. Our assessment informed by stakeholder responses to both the October 2021 and March 2023 consultations – is that the implementation of the Clean Heat Market Mechanism will not disadvantage or have disproportionately negative direct impacts on any groups sharing protected characteristics. The main intended outcomes of the policy - carbon emissions reductions and strategic alignment to net zero objectives - are non-excludable public goods and therefore expected to benefit the majority of the population without distributional impacts for specific groups.

- The cost of a low carbon heating system may remain a barrier to those on lower incomes, which may limit the direct benefits of the policy for this group. While lower household income is not itself a protected characteristic, it is often correlated with several protected characteristics, such as certain race and disability groups.  It is also therefore possible the policy will have higher indirect impacts for certain such groups with respect to the balance of near-term costs and benefits. However, we believe that there is strong justification for proceeding with the policy despite this potential disparity in the timing of the indirect accrual of costs and benefits to protected groups, and that it is proportionate to proceed. Without policies such as this to drive market scale and promote long-term cost reduction, costs and prices of low-carbon heating are less likely to reduce over time, meaning that the cost of switching to low-carbon heating will remain prohibitive for more households for longer.

- We will continue to keep the direct and indirect impact, or potential impact, on groups with protected characteristics and on wider inequalities under review during the scheme’s implementation and during consideration of any potential changes to the scheme’s design in the future.

- With respect to the second and third arms of the Equality Duty, while our assessment is that the policy will not have significant immediate impacts on either advancing equality of opportunity or fostering better relations between groups that share protected characteristics and those that do not, we believe that contributing to decreasing the cost of home decarbonisation and of energy-efficient appliances such as heat pumps through helping scale these markets and supply chains may have a minor role in reducing a current income- and/or wealth-based disparity of access to these technologies. This present disparity may disproportionately affect those in certain groups that share protected characteristics. As wider incentives start to further encourage decarbonisation, addressing this disparity, through policies such as this and others, will become increasingly important, in order to maintain equality of opportunity and preserves social cohesion and good relations between groups.

# Impact on Business

- The impact of the Clean Heat Market Mechanism on businesses, both in the heating appliance sector and more broadly, will depend upon the wider policy framework for heat decarbonisation of which it is part as well as wider market developments, which will be continuing as this policy is implemented. The analysis that follows reflects our current central assumptions regarding the future costs of heat pumps and plans for other policies such as the Boiler Upgrade Scheme.

- The obligation under the Clean Heat Market Mechanism applies to the manufacturers of fossil fuel heating appliances sold in the UK. The supply chain will also be affected by the obligation, as third parties might be involved in delivering the installations to meet the targets. However, in line with Better Regulation Executive guidance, these changes are defined as ‘resources used to comply with regulation’ and so their impacts are not captured in the EANDCB. A qualitative assessment of the impact on installers is included below.

- Fossil fuel heating appliance manufacturing is a relatively concentrated market sector, with four companies responsible for around 91% of annual gas boiler sales (ca. 1.5m per year), and five companies responsible for around a further ~7% of sales. The four largest companies, and a number of the second-tier companies, are all multi-technology corporations producing and selling a range of appliances including heat pumps in several countries. In the oil boiler sector, four companies share around 93% of annual sales (around 55,000 per year), and another two companies a further 3%. The largest four also produce heat pumps. The following sections provide illustrative estimates of the main costs that these businesses might face.

- Companies with boiler sales less than 20,000 relevant gas boilers and 1,000 relevant oil boilers, corresponding to roughly 1% of the UK market are exempt. Companies that meet the criteria of the small companies regime in the Companies Act 2006 are also not in scope of the obligation. These minimum thresholds and the exclusion of small companies limit the number of obligated companies and limit the costs to smaller firms or those with only limited activities in the UK. It also ensures that the administrative complexity and costs of administering the obligation are kept proportionate.

## Familiarisation and admin costs

- In delivering their obligation, businesses will incur familiarisation and administrative costs. These will vary by business - depending on factors such as their setup, size and decisions on how to comply with the obligation - but are likely to include items such as the cost of running IT databases, staff time, data verification and reporting to the scheme administrator. They may also include indirect costs, such as human resources and legal costs.

- We estimate familiarisation costs at the start of the scheme of around £48,000 for the policy in total.

- There are also likely to be administrative costs to these businesses associated with compliance with the scheme once in force, for instance from reporting and the provision of evidence of compliance, participation in audit processes, etc. Internal estimates, and comparisons with comparable schemes, produce an indicative high estimate of around £8m per year across all obligated parties.  As part of the monitoring and evaluation strategy below, DESNZ will commission a process evaluation which will gather evidence on these costs to inform both more accurate assessments of this policy and to contribute to assumptions for assessments of other potential policies and schemes where similar.

- Table 7: Annual Admin and Familiarisation Costs

| £m, PV - 2024 prices, 2024 base year | 2025/26 | 2026/27 | 2027/28 | 2028/29 |
| --- | --- | --- | --- | --- |
| Familiarisation Costs | <£0.1m | £0m | £0m | £0m |
| Admin Costs | £7.6m | £7.3m | £7.1m | £6.8m |
| Total | £7.6m | £7.3m | £7.1m | £6.8m |

## Obligation costs

- The cost to businesses of meeting their CHMM obligations are difficult to assess with confidence. At the lower end of the range, they could amount only to the familiarisation and administrative costs estimated in the section above. However, the difference in cost between heat pump and fossil fuel systems and the presence of market barriers and failures means that there are scenarios where businesses may need to subsidise heat pumps to induce households to install them. For each installation, the size of such subsidy required would be affected by several factors:

  - The speed at which heat pump costs fall relative to fossil fuel systems as the market develops.

  - Consumer familiarity and willingness to pay a premium for heat pump installations (over boiler replacements) and how these changes over time.

  - The extent to which other policies supporting the deployment of heat pumps reduce costs. This could be directly with subsidy such as the Boiler Upgrade Scheme and Warm Homes Social Housing Decarbonisation Fund. Other policies, including financial support for training and/or manufacturing will also lower costs to businesses.

  - The difference in annual running costs for customers between heat pumps and fossil fuel systems. This is determined by retail energy prices and associated policy.

  - The development of the market for installation credits and the extent to which businesses wish to use these to meet their obligations.

  - The extent to which higher revenues from heat pump sales and increased market share are of value to businesses.

- The possibility to trade credits means that the costs faced by different obligated parties could vary significantly, depending on whether they decide to meet their credit obligation, in part or in whole, through acquiring credits from other heat pump manufacturers. Some of the costs that obligated parties will face are transfers between businesses; others will be offset by direct benefits to business. In line with Better Regulation Executive guidance, we have excluded these costs from the total.

- In the least-cost scenario for obligated parties, heat pumps would either be cost-equivalent with fossil fuel technologies, or a combination of consumers' willingness to pay and/or policy subsidy would be sufficient to cover the difference in cost.

- Obligated parties have the option to make a ‘payment in lieu’ instead of selling a heat pump at a cost of £500. If the costs of subsidising heat pumps to achieve the necessary sales is too high, they may prefer to make these payments. We can therefore consider the maximum cost to business as the total number of heat pump sales attributed to the CHMM multiplied by this £500 payment amount, plus the familiarisation and admin costs described above. These costs are estimated in table 12 below.

- Should boiler manufacturing businesses decide to meet the obligation by selling heat pumps themselves, they may need to subsidise these heat pumps to encourage households to install them. This subsidy (delivery cost), if present, would be a direct cost to business. We estimate that this cost to business is between our minimum and maximum estimates described above. All the additional costs associated with manufacturing, marketing and installing the heat pumps over and above the equivalents for boilers should be considered direct costs associated with the scheme; however, they will be (fully or partly) offset by the direct revenues generated from the heat pump sales. This means that the only true cost to business would be the possible extra subsidy they might need to offer to persuade some households to install heat pumps.

- The development of the market could significantly reduce these delivery costs, for example, if consumers were presented with affordable options for multi-year financing, this might increase their demand for and willingness and ability to pay towards heat pump installations.

- Within our analysis there is an assumption that consumers’ willingess to contribute towards the cost of their heat pump installation will reduce the cost for obligated parties by £2500 per installation on average. This is an illustrative estimate and is based in part on evidence from the Boiler Upgrade Scheme to date, which suggests that applicants are willing to contribute, on average, around £8000 for the total cost of the heat pump (and therefore over £5000 on top of the cost of an average boiler). However, these applicants to the scheme to date may be considered early adopters with a particular interest in decarbonisation. In future years, one might expect the average willingness to pay to decrease as deployment increases; on the other hand, as the market develops, it is likely there will be increased consumer awareness as well as new financing arrangements that could combine to make it easier to find a greater number of willing-and-able-to-pay heat pump consumers.

- Consumer demand, as well as all the other factors affecting the subsidy cost listed above, must be considered when setting the level of the payment in lieu. The level of the payment in lieu will be kept under review throughout the lifetime of the policy, to ensure that it provides an incentive for obligated parties to sell heat pumps and/or purchase equivalent credits.

Table 8: Obligation Costs

| 2024 prices, Total Cost £ | Option 1 |
| --- | --- |
| Minimum Possible Cost – No additional cost above Familiarisation and Admin | £0m |
| Maximum possible cost – Estimated subsidy cost based on conservative assumptions | £8m |

- Following our approach above, we have calculated the obligation costs with respect to the Scheme Regulations 2024. Given our assumptions regarding support from other heat pump policies and current potential contributions from consumers, under Option 1, in our central scenario, the subsidy cost to business would be negligible. However, there is significant uncertainty around this analysis. For caution, under Option 1 we use the maximum possible average annual cost (£8m) for our overall cost to business calculation.

## Direct costs to businesses

- Combining the familiarisation and administrative costs with the estimated obligation costs described above gives the following estimates of total cost to business.

Table 9: Total Direct Cost to Business

| 2024 prices, Total Cost £ | Option 1 |
| --- | --- |
| Minimum Possible Cost – No additional cost above familiarisation and admin | £29m |
| Maximum –A payment in lieu for all heat pumps attributed to the CHMM plus familiarisation and admin | £37m |

Table 10: Annual Direct Cost to Business

| 2024 prices, Annual Cost £ | 2025/26 | 2026/27 | 2027/28 | 2028/29 | Total |
| --- | --- | --- | --- | --- | --- |
| Familiarisation Costs | £0.05m | £0m | £0m | £0m | £0.05m |
| Admin Costs | £7.6m | £7.3m | £7.1m | £6.8m | £7.6m |
| Maximum Payment-in-Lieu | £8.3m | £0m | £0m | £0m | £8m |
| Total Maximum possible cost – A payment in lieu for all heat pumps attributed to the CHMM plus familiarisation and admin | £15.8m | £7.3m | £7.1m | £6.8m | £37.1m |

- Our estimate for direct costs to business reflects the Scheme Regulations 2025 as introduced and conservatively takes our maximum estimates of cost.

- Actual cost to business may be lower than this maximum estimate: more ambitious heat pump cost reductions, in line with the Government’s ambition for parity of lifetime costs by 2030, the increased appeal of heat pumps to consumers as the market grows, and the impact on the market of complementary wider policy would decrease the cost significantly. Businesses may choose to pass through costs to other parts of the business and/or the prices of other products in their portfolio, although alternative approaches to funding market expansion may be available. These potential cost reductions could increase the likelihood of businesses meeting the obligation to install heat pumps instead of making a payment in lieu.  We will monitor and assess market impacts of the scheme as part of our approach to monitoring and evaluation. Table 14 below summarises the key costs to business.

Table 11: Summary of Cost to Business

| 2024prices, Total Cost £ | Description | Key data sources and uncertainty/robustness | Option 1 |
| --- | --- | --- | --- |
| Familiarisation Costs | The cost to understand the obligation, based on the value of staff time | Based on illustrative estimates of 100 hours needed for familiarisation for the 13 companies assumed to be in scope. We used the hourly wage for management consultants and business analysts (~£28/hr) from the ONS, uplifted with the non-wage cost uplift from RPC / Eurostat 2019 (21.78%) | £48,000 |
| Administrative Costs | Costs associated with compliance with the scheme once in force, for instance from reporting and the provision of evidence of compliance subject to the final design of administrative processes, participation in audit processes, etc | Assuming a cost of £0.5m for the 13 obligated company (based on the 2014 Renewable Transport Fuel Obligation post-implementation review estimate, which includes the cost of reporting, trading, verification, policy, audit, marketing, general management and technical issues) | £29m |
| Obligation Costs | The costs that obligated parties may need to incur to support households to install heat pumps. | When producing our upper estimate of obligation costs, we assume a payment in lieu for all heat pumps attributed to the CHMM. | £8m (£0m – £8m) |
| Net Costs – Range |  |  | (£30m – £37m) |
| Net Costs – Central Estimate |  |  | £37m |
| Equivalent Annualised Direct Cost to Business (EANDCB) |  |  | £9.5m |

- As mentioned above, there is likely to be variation in the net impact on different businesses in this sector, with the possibility that certain businesses face a net cost and others a net benefit, through benefitting from trading surplus credits under the mechanism.

## Indirect costs to businesses

- The policy should have a significant positive impact for installers and maintenance engineers of the low-carbon heating appliances in scope, many of which are small or micro enterprises, through increased business activity. There are currently over 1,700 businesses in the UK certified with the Microgeneration Certification Scheme (MCS) to install heat pumps, estimated to employ over 7,000 installers. The total number of trained heat pump installers is, however, likely to be greater than this, as MCS Certification is only required for installations receiving Government grant funding. The increased level of deployment driven by the policy should increase the revenues of these businesses; furthermore, by providing certainty in the growth of the heat pump market, the policy should encourage businesses to invest in installer training, with a positive impact on the supply chain. To further support this, Government announced a £5m Heat Training Grant to support trainees in England taking training relevant to heat pumps and heat networks. This is expected to support 10,000 training opportunities up to 2025, including at least 6,000 for heat pumps.

- Installers are already required to notify heat pump installations to receive Government funding in England, Wales and Scotland, as well as to demonstrate compliance with building regulations. Therefore, the additional administrative burden on installers associated with the Clean Heat Market Mechanism is expected to be minimal.

- Any decline in future fossil fuel boiler sales should be matched by an increase in heat pump sales, and we expect this to lead to an overall increase in work for the heating workforce. Heating engineers who choose not to diversify into the low-carbon market could see a potential negative impact, but we expect this impact to be small. We expect the negative impact to be small. Recent Government research found that around three quarters of heating and cooling installers could either install heat pumps already or are open to being persuaded to train to install heat pumps in the long term. We expect the majority of heat pump installers in 2028 to be current heating engineers who train to install heat pumps alongside other heating technologies; those not willing to retrain are likely to be planning to retire within the timescales of the CHMM and therefore will not be negatively impacted. Overall, Government expects the installer workforce to expand, creating new opportunities for apprentices. A new three-year Low Carbon Heating Technician apprenticeship launched in September 2023, and the existing Plumbing & Domestic Heating Technician apprenticeship is being revised to ensure that all heating apprentices learn the core skills needed for low-carbon heating.

- The policy is likely also to have a positive impact for other market actors involved in the promotion, retail, financing or installation of heat pumps, of ancillary equipment such as thermal storage, or of related services, for instance in relation to demand-side response. This may include certain energy suppliers and energy service companies, for instance.

- Regarding impacts on businesses more broadly, i.e., outside the heating market and wider supply chain, while direct impacts are likely to be limited, it is likely that over time there will be an indirect benefit from reduced costs of low-carbon heating appliances and their installation, reducing the costs of switching to appliances that consume less energy and of compliance with present and future minimum energy efficiency standards and building regulations. Being indirect, such benefits don’t feature in the EANDCB assessment calculation for the policy.

# Competition and Trade Impact

- We do not assess that the policy will lead to significant negative impacts on competition in terms of range of suppliers in the market, suppliers’ ability to compete, suppliers’ incentives to compete vigorously, or the choice and information available to consumers. Rather, our assessment is that the fossil fuel appliance market will remain competitive and that the low-carbon heating appliance market is likely to become larger, with more actors and more pressure to compete vigorously on costs and other differentiators, with benefits for consumers. The ability to trade in credits on the secondary market, target carry-forward and de minimis conditions are also expected to limit the risk of a negative impact on competition.

- The policy applies equally to the manufacturers of fossil fuel heating appliances imported to the UK market as to those manufactured in the UK, thereby ensuring that competition and trade are not distorted in favour of overseas competitors. The policy could in principle deter new entry of boiler manufacturers to the UK market. In practice, however, the UK boiler market is highly mature and significant new entrants in the absence of this policy, while not impossible, would perhaps therefore be unlikely.

- By providing a market signal of a firm and fast-growing UK heat pump market, the policy is likely to increase the attractiveness of the UK market to international actors in the heat pump market, either to begin or expand import of their products or services and/or to consider investing in establishing a UK presence, potentially including manufacturing. To support this, the government launched a £30 million Heat Pump Investment Accelerator Competition (HPIAC) in 2023. The Competition awards grant funding for eligible capital costs relating to the construction of new factories, or the expansion or retooling of existing factories, to produce heat pumps or their components. Attributing any increase in inward investment in the UK heat pump market solely to the Clean Heat Market Mechanism, rather than to a combination of policy and market drivers, would though be very difficult.

- Exports of fossil fuel heating appliances are not in scope of the policy and so are not expected to be affected; on the other hand, the policy may contribute to a modest positive impact on exports of heat pumps if it contributes to e.g. investment decisions to locate heat pump manufacturing capacity in the UK (and subsequent export) and/or the development in the UK of exportable know-how, product innovations or successful ancillary services related to the heat pump market.

# Small and Micro Business Assessment (SaMBA)

- Micro businesses do not directly incur costs as a result of the scheme regulations. The regulations establish de minimis threshold exclusions from the policy scope: manufacturers of fossil fuel appliances with less than 20,000 relevant gas boiler sales or less than 1,000 relevant oil boiler sales, currently equivalent to a little over a 1% market share in each case, are not in scope of the obligation to meet heat pump targets with respect to those appliances. Companies that meet the criteria of the small companies regime under the Companies Act 2006 are also not in scope of the obligation. These minimum thresholds and the exclusion of small companies limit the number of obligated companies and limit the costs to smaller firms or those with only limited activities in the UK.

- The regulations establish that the administration of the CHMM will be based on corporate groups: related parties, business units, or brands within the same corporate group are treated as one ‘appliance manufacturer’ entity for the purposes of determining targets under the scheme and awarding heat pump credits. The obligation applies to all manufacturers of relevant fossil fuel appliances sold for installation in the UK, who are not excluded as per above, regardless of whether the manufacturing takes place in the UK or of whether the company has a UK corporate presence. This approach aims to ensure the obligation is applied fairly and does not have a disproportionate impact on UK manufacturers relative to non-UK manufacturers. Therefore, businesses with few or zero UK employees whose levels of UK sales are above the de minimis threshold will be considered in scope of the obligation. These businesses may class as small businesses in terms of numbers of UK employees (i.e. fewer than 49), but are generally larger entities in global terms, with higher staff numbers in their headquarter and/or manufacturing locations.

- It is possible that smaller businesses in scope of the obligation – for instance those with just over 1% of market share (around five in total at present) – may face proportionately higher costs compared to larger businesses in scope. For instance, the opportunity costs of upfront familiarisation with requirements and ongoing administration/reporting may be higher for a firm without dedicated compliance or regulatory affairs staff. However, it should be noted that any businesses in scope have the option of purchasing credits from other heat pump manufacturers, reducing transition and other fixed costs for such firms.

- Outside of parties in scope of the obligation, a substantial number of existing, diversifying or new micro and small businesses engaged in the installation of low-carbon heating are likely to benefit indirectly from the policy’s increase in business in this area. Data from the Heat Pump Association indicates that the large majority of existing installers are small or micro businesses. Since a key objective of the Clean Heat Market Mechanism is to build the supply chain, we expect the scheme to maintain existing small or micro low carbon heat businesses and create opportunities for growth and new businesses in this segment to emerge. There are currently over 7,000 such installers in the UK, but there is expected to be demand for around 33,000 by 2028, according to the Heat Pump Association.

# Monitoring and Evaluation

## Aims and Objectives

- Given the scope and potential impact of the CHMM, a robust monitoring and evaluation approach will be implemented. The monitoring and evaluation will demonstrate the outcomes and impact of the proposed scheme, providing a measure of success against the policy objectives described above. It will also yield evidence to inform future low-carbon heat policy development. Undertaking ongoing M&E throughout the lifetime of the CHMM will support the identification of risks and highlight areas for improvement, allowing for in-flight policy changes.

- The aims of the monitoring are to:

  - Support regular reporting of the realisation of scheme outcomes and benefits.

  - Yield monitoring data which can be utilised alongside evaluation to assess whether the CHMM met its objectives.

  - Support scheme management by tracking of delivery and amendments to the scheme.

  - Alongside evaluation, identify barriers and opportunities experienced during delivery to support the design of future policies.

- The aims of the evaluation are yet to be defined as an evaluation plan is currently under development, subject to a decision on whether to evaluate the CHMM individually or in partnership with other policies as part of an Electrification of Heat Programme (EoH) evaluation. In any case, the evaluation will provide an assessment of whether the CHMM realised it’s intended impacts, in order to provide an independent assessment of whether the scheme was a success, to support accountability and to produce learnings for future policy development. The aims of an evaluation could be to:

- Assess the extent to which realised outcomes and impacts were attributable to the CHMM/EoH Programme.

- Consider whether the CHMM/EoH Programme supported the realisation of wider impacts or had unforeseen impacts.

- Alongside monitoring, identify barriers and opportunities experienced during delivery to support the design of future policies.

- Assess whether the CHMM/EoH Programme constituted value-for-money.

- Provide an evidence base to support scheme communications.

## Monitoring metrics

- The scheme administrator is required to collect detailed monitoring data and this will be combined with DESNZ data to provide regular reports. This data will be used to ensure close monitoring of the realisation of outcomes and benefits of the CHMM. The expected key scheme metrics are set out in Table 16 below.

Table 12: Monitoring Metrics

| Metric | Unit | Source | Frequency |
| --- | --- | --- | --- |
| Delivery |  |  |  |
| UK retrofit heat pump sales | # | Scheme administrator data derived from installer notifications | Monthly |
| of which hybrid | # | Scheme administrator data derived from installer notifications | Monthly |
| of which replacing off-gas-grid fossil fuel heating | # | Scheme administrator data derived from installer notifications | Monthly |
| UK fossil fuel boiler sales | # | Scheme administrator data derived from boiler manufacturer reporting | Quarterly |
| Total number of HP credits presented vs total obligation for year (i.e. shortfall) | # | Scheme administrator data | Annually |
| Number of penalties issued | # | Scheme administrator data | Annually |
| Benefits Tracking |  |  |  |
| Carbon abatement | MtCO2e | Scheme administrator will collect information on numbers and types of heat pump installed, allowing to estimate the carbon savings from displacing fossil fuel heating systems. | Quarterly |
| Renewable heat produced | MWh | Scheme administrator will collect information on the heat pumps installed, allowing estimates of renewable heat generation to be produced. | Quarterly |
| Improvements in air quality | £ | An estimate of air quality impacts can be derived from heat pump deployment numbers, using the methodology provided in Green Book supplementary guidance. | Quarterly |
| Cost of heat pumps installed | £ | MCS data | Quarterly |
| Cost-effectiveness | £ per MtCO2e abated and £ per TWh renewable heat generated | Calculated using the total cost of hp installations, the total carbon abated, and the total renewable heat generated. | Mid-scheme / end of scheme reviews |

## Evaluation

- Arrangements for the evaluation of the CHMM are currently subject to a decision as to whether it will receive its own policy level evaluation or be evaluated as part of a wider evaluation of the EoH Programme in which it sits.

- The benefits of a programme evaluation are that it is better placed to assess shared impacts where individual policy evaluations struggle to attribute impact to a particular scheme in a busy policy environment where multiple mechanisms are attempting to stimulate the same results. However, this comes at the expense of a greater volume of policy specific evidence, particularly in the process evaluation, which is better served by individual policy evaluations.

- In this absence of this decision, this section will provide assurances that all relevant considerations are being taken into account in designing an appropriate evaluation and provide some insight into what either evaluation approach could look like. It also seeks to assure readers that at least a minimum of evaluation activity will be undertaken to assess the CHMM or the EoH Programme in which it resides, against their respective objectives.

### Approach & Methodology

- Any design would see the evaluation providing an assessment of whether the CHMM realised it’s intended impacts (benefits and wider impacts), drawing on secondary monitoring data and primary data collection, to support the aims of its impact evaluation component. The impact evaluation is likely to employ quasi-experimental impact analysis, which will be scoped in partnership with existing academic experts. This may be supplemented by theory-based approaches, such as contribution analysis, where particular impacts do not lend themselves to quasi-experimental approaches.

- Whether realised impacts constituted value-for-money will also be part of the evaluation. Social cost-benefit analysis synthesised alongside wider evaluation evidence in a 3E’s approach is likely to be the preferred method of economic evaluation. An existing economics consultant will support the design of this work.

- Scheme processes or wider programme delivery for a policy or programme evaluation respectively, and the extent to which barriers and enablers in these respects facilitated the realisation of impacts, will be the subject of a process evaluation.

- Any evaluation design will inevitably provide various avenues for primary data collection, using qualitative methods and/or surveys. Though these will principally provide data to support an assessment of the extent to which the CHMM met its objectives, there will also be an opportunity to pursue wider areas of inquiry relevant to the CHMM and Clean Heat policy more broadly.

### Evaluation delivery, outputs and timelines

- It is most likely that this evaluation will adopt a hybrid delivery approach, whereby responsibility for undertaking the evaluation is split between DESNZ analysts and external evaluation suppliers. This approach would see external evaluation suppliers collecting primary data, leading on the majority of analysis and producing outputs, to ensure the impartiality of this work. The supplier will be recruited through a fair and open competition and will be assessed based on price and the quality of their bid for this project. However, where the skills of internal DESNZ analysis can be utilised, such as in undertaking basic statistical analysis of monitoring data or producing cost-benefit analysis, they will undertake this work, subject to oversight from the supplier, to ensure value-for-money.

- The supplier will produce a range of outputs, which may take the form of reports, dashboards and data tables, containing the findings of the evaluation.

- The findings from any evaluation will be synthesised annually and contained in a published annual interim report. A final synthesis report will be published at the end of the evaluation.

- Findings from the evaluation will be used to support the design and implementation of any future scheme which seeks to support the uptake of low carbon heating and will generate transferable knowledge which is also relevant in respect of other energy efficiency measures, and/or market-based policy interventions in other sectors. The evaluation will also seek to explore the emergence of any perverse incentives or unforeseen impacts of the CHMM.

- Any evaluation will run from 2025 to 2029. In the case of a CHMM policy evaluation, this will ensure that any future years of the scheme post-2025 are subject to an evaluation, whereas for a programme evaluation, this means it will conclude at a point when the constituent policies in the programme are all ending.

- Appropriate budget and resources are being sought within DESNZ to deliver the evaluation.

# Annex – full list of modelling assumptions and risks

## General assumptions

Table 13: Appliances Characteristics in Central Scenario*

|  | Capex (Average Price Today Excl. VAT) | Capex Reduction By 2028 | Average Annual Efficiency | Lifetime (Years) |
| --- | --- | --- | --- | --- |
| Heat Pump | See Table 17 below | 20% | 280% | 18 |
| Hybrid Heat Pump (Split) | £11,600 | 20% | As above for the heat pump component; 84% for the gas boiler component in 2024; 89% from 2025 onwards. Heat Pump/Gas split equiv. to 65%-35%. | 18 |
| Gas Boilers | £2,700 | – | 84% in 2024, 89% from 2025. | 15 |
| Oil Boilers | £4,600 | – | 84% | 15 |
| Coal Boilers | £4,600 | – | 72% | 16 |
| Storage heaters | £3,700 | – | 100% | 20 |

*Maintenance costs are assumed to be equal in all scenarios

### Table 14: Heat demand and HP Capex costs of households installing HPs

|  | Households on the gas grid (OnGG) - Average | Households heated by direct electric appliances | Households off the gas grid (OffGG) - average |
| --- | --- | --- | --- |
| Average heat demand (kWh) | 9,900 kWh | 7,200 kWh | 13,600 kWh |
| Space heating demand increase after HP installation | 10% | 10% | 10% |
| Capex costs of installing a HP (average price today) | £11,600 | £11,500 | £13,000 |

## Sensitivity analysis assumptions

S1: Energy prices

Low and high fuel price projections come from the HMT Green Book supplementary guidance.

S2: Carbon prices

Low and high carbon values series comes from the HMT Green Book supplementary guidance.

S3: Efficiency of heating system

We tested a low and a high scenario, with average heat pumps efficiencies of 253% and 309%, respectively. These values represent the 25th and 75th percentile of data from the Electrification of Heat Demonstration Project.

S4: Future capital cost reduction for heat pumps – Low/High

In this sensitivity analysis we explore a scenario with 50% cost reductions by 2030. This assumption is dependent on innovation in the equipment as well as economies of scale benefits in heat pump installations. We have also assumed a lower cost reduction scenario with a 10% cost reduction by 2030. In both scenarios we have assumed a linear cost reduction from 2025 to 2030.

S5: Low/High hybrid deployment

In the high end of this scenario, we assume that half of the On Gas-grid market mechanism additional installations are hybrid heat pumps, in which a heat pump works together with a gas boiler. We also assume a low scenario where no hybrids are deployed. In all scenarios, we assume the heat pump component to meet 65% of the heat demand with the gas boiler meeting the remaining 35%.

S6: Target carry-forward

In this scenario we assume that firms are allowed to carry forward 35% of their deployment credits into the next year. This results in year 1 heat pump deployment being 65% of the year 1 target and year 2 deployment being 65% of the year two target, plus the remaining 35% of year one deployment that was carried over. The Regulations don’t have carryover allowance from FY 28/29, the final year of the assessed period, so we have assumed that no heat pumps can be carried forward in this year.

2 MCS Installation Database. NB MCS data here may not represent the full number of heat pump retrofits in 2023, but is expected to represent the majority.

3BSRIA UK Domestic Boilers (2024)

5DESNZ (2022), Winter public attitudes tracker, see Figure 2.1 https://assets.publishing.service.gov.uk/government/uploads/system/uploads/attachment_data/file/1140686/BEIS_PAT_Winter_2022_Heat_and_Energy_in_the_Home.pdf

BSRIA (2024), ‘Domestic boilers market analysis 2023‘

DESNZ (2023), Winter public attitudes tracker - See Figure 2.1 https://assets.publishing.service.gov.uk/media/65fc5d5e65ca2f001b7da82c/DESNZ_Public_Attitudes_Tracker_Winter_2023_Heat_and_Energy_in_the_Home__Revised_.pdf

BEIS (2019), RHI evaluation interim report

https://www.gov.uk/government/publications/rhi-evaluation-interim-report-applicant-reaction-to-reform-announcements

DESNZ (2024), Boiler Upgrade Scheme’s, August statistics – See Table 1.4* https://assets.publishing.service.gov.uk/media/66f29e3b7da73f171776408d/Boiler_Upgrade_Scheme__BUS__Statistics_August_2024.xlsx

*For simplicity, we merged Oil, LPG and Coal fuel households into a combined Fossil Fuel off gas grid total and we excluded installations whose fuel type displaced is “None”, “Other” or “Unknown”.

In Scenario 6: Target carry-forward we have not considered a ‘low’ scenario as there is no low variant of the sensitivity, relative to the central assumption of zero carry-forward.

BEIS (2023), Green Book supplementary guidance, https://www.gov.uk/government/publications/valuation-of-energy-use-and-greenhouse-gas-emissions-for-appraisal

BSRIA (2024), ‘Domestic Boilers Market Analysis 2023’

This is based on the illustrative assumption of 100 hours needed for familiarisation with the scheme and 13 companies in scope of the obligation. We have used the hourly wage for management consultants and business analysts (~£28/hr) from the ONS ASHE data, uplifted with the non-wage cost uplift from RPC / Eurostat 2019 (21.78%)

Using an estimate of £0.5m admin cost per year, based on the Renewable Transport Fuel Obligation post-implementation review,  and assuming 13 companies will be in scope with the obligation. Alternative estimates, e.g. based on other comparable schemes, are lower, so our central scenario is likely to overestimate costs.

Max Obligation costs calculated in Table 12 assumes that obligated parties do not deploy any of the 16,000 additional heat pumps required to meet the 2025/26 CHMM target and opt for a £500 payment-in-lieu per missed heat pump.

BEIS (2023), Heating and cooling installer study,

https://www.gov.uk/government/publications/heating-and-cooling-installer-study

Companies Act (2006), https://www.legislation.gov.uk/ukpga/2006/46/part/15/chapter/1/crossheading/companies-subject-to-the-small-companies-regime

Heat Pump Association (2019), Installer survey results,

https://www.heatpumps.org.uk/wp-content/uploads/2019/11/Installer-Skills-Survey-Summary.pdf

Heat Pump Association (2023), https://www.heatpumps.org.uk/about/installers/

The EoH Programme comprises the Boiler Upgrade Scheme, Heat Pump Investment Accelerator Competition and other policies, in addition to the CHMM.

DESNZ’ analysis of the National Housing Model results.

Catapult Electrification of Heat Interim Report Table 8.1 - https://es.catapult.org.uk/report/electrification-of-heat-interim-heat-pump-performance-data-analysis-report

Existing performance based on In-situ monitoring of condensing boilers report, future performance on the 2022 consultation to improve boiler standards and efficiency.

https://assets.publishing.service.gov.uk/government/uploads/system/uploads/attachment_data/file/933099/burning-wood-consult-bsria-report1.pdf

Off gas grid households are assumed to all use oil boilers for heating.

DESNZ’ analysis of the National Housing Model results. Includes space and hot water heating.

UCL (2017), https://discovery.ucl.ac.uk/id/eprint/1566603/

DESNZ’ analysis of the National Housing Model results. Heat Pumps capex costs are for air source, low temperature heat pumps. This includes the cost of the unit, fixtures, buffer tank and hot water cylinder, controls, labour and upgrade to radiators.

Including the cost of installing a wet distribution system. Households currently heated by direct electric appliances tend to be smaller than average which partly offsets the increase in price due to the installation of a wet distribution system.

BEIS (2023), Green Book supplementary guidance, https://www.gov.uk/government/publications/valuation-of-energy-use-and-greenhouse-gas-emissions-for-appraisal

BEIS (2023), Green Book supplementary guidance, https://www.gov.uk/government/publications/valuation-of-energy-use-and-greenhouse-gas-emissions-for-appraisal

Catapult Electrification of Heat Interim Report Table 8.1 - https://es.catapult.org.uk/report/electrification-of-heat-interim-heat-pump-performance-data-analysis-report